# BlackRockIoT communication protocol and data format ( wired series )

Modification Record

| Versio n | Modify location | Modification Notes | Modified by | Modificati on Date |
| --- | --- | --- | --- | --- |
|  |  | Create initial documentation | BlackRockIoT | 202 2 - 05 - 15 |
|  |  | 1: Added 94-01 ADC analog upload protocol 2: Input2 uses 94-05 Bit10 to upload | BlackRockIoT | 2024-10-16 |
|  |  | Alarm protocol flag optimization and confirmation | BlackRockIoT | 20241127 |
|  |  | Added fake base station alarm and GPS interference alarm | BlackRockIoT | 25050303 |
|  |  | 9B protocol adds ibutton type | BlackRockIoT | 20250402 |
|  |  | Added 94-02 03 altitude agreement | BlackRockIoT | 20250311 |
|  |  | Added 94-03 Bluetooth oil sensing protocol | BlackRockIoT | 20250618 |
|  |  | When adding 94 type to 06 (temperature and humidity integrated sensor) | BlackRockIoT | 20250715 |
|  |  | Added 94 type 07 tire pressure sensor protocol | BlackRockIoT | 20250813 |
|  |  | Newly added 0X4C air conditioner on alarm\ 0X4D air conditioner off alarm | BlackRockIoT | 20250918 |

Models applicable to the agreement : ( Model of the device imported into the platform )

BRO1

Table of contents

- PROTOCOL PACKAGE FORMAT	4

Detailed explanation of protocol number	4

- PROTOCOL PACKET DECOMPOSITION	5

  - Login Pack	5

    - Login package [ 0X01 ]	5

    - Login packet reply (server reply)	7

  - Heartbeat packet [0X13]	8

    - The terminal sends a heartbeat packet	8

- Server heartbeat packet reply	9

  - GPS POSITIONING PACKAGE [0X22]	11

    - The terminal sends a positioning packet	11

    - Server location packet reply	14

  - LBS multi-base station extended information package [0X28]	15

    - The terminal sends LBS multi-base station package	15

    - The server LBS multi-base station packet replies	16

  - Alarm package [0X26]	17

- The terminal sends an alarm packet	17

- Server alarm packet reply	22

  - Online instruction [0X80]	29

    - Server online command sent	29

    - Terminal online command reply	29

  - Time calibration package [0X8A]	31

- The terminal sends a time calibration request	31

- The server responds with time information	31

  - Information transmission general package [0X94]	33

- The terminal sends a general information transmission packet	33

  - Type is 	33

  - Type is	36

  - Type is 	39

  - Type 0 D (Oil sensor) (supported by BR1201) 	43

  - When the 94 type is 1E ( temperature sensor ) (supported by BR1201) 	44

  - When the 94 type is 1F ( oil level sensor ) (supported by BR1201) 	45

  - External module transparent transmission protocol [0X9B]	47

The server sends a data packet to module	47

The module replies data to the server	48

Special protocol: This protocol is 9B, but it is uploaded from the terminal to the server	48

  - External module transparent transmission protocol (module sending) [0X9C]	50

The module sends data packets to the server	50

The server responds to module	50

------The following is the reserved agreement: (not currently used)	57

Differentiated login package [0X01]	57

Differentiated positioning package [0X12]	58

Differentiated LBS package [0X18]	60

Differentiated alarm package [0X16]	62

Differentiated command reply packet [0X15]	65

- WIFI INFORMATION PACKAGE [0X2C]	67

  - The terminal sends WiFi packet	67

  - Server WIFI packet reply	68

- Alarm package (LBS) [0X19] (vehicle-mounted and vehicle-mounted OBD do not have this package)	24

  - The terminal sends an alarm packet	24

  - Server alarm packet reply	28

- GPS ADDRESS REQUEST PACKET [0X1A]	69

  - Terminal address request packet	69

  - Server address request packet Chinese reply	69

  - Server address request packet English reply	70

- LBS ADDRESS REQUEST PACKET [0X17]	72

- Terminal address request packet	72

- Server address request packet Chinese reply	72

- Server address request packet English reply	73

- INFORMATION TRANSMISSION PACKAGE [0X98] ( 98 PACKAGE, ONLY USED IN SOME PROJECTS ) 	75

  - The terminal sends to the server	75

  - The server replies to terminal	76

- 4G GPS positioning package (4G base station, protocol number: 0xA0)	77

  - The terminal sends a positioning packet	77

  - Server location packet reply	79

- 4G LBS multi-base station extended information packet (4G base station, protocol number: 0x A 1)	80

- The terminal sends LBS multi-base station package	80

- Server LBS multi-base station packet reply	81

- 4G single fence alarm package (4G base station, protocol number: 0xA3 / 0xA4 )	82

  - The terminal sends an alarm packet	82

  - Server alarm packet reply	86

  - The server alarm packet Chinese address reply	86

  - Server alarm packet English address reply	87

- 4G LBS alarm package (4G base station, protocol number: 0xA5)	88

  - The terminal sends an alarm packet	88

- ANNEX	94

  - Appendix 1 CRC-ITU Lookup Algorithm C Language Code Snippet	94

  - Appendix 2 Business Flowchart	95

## Protocol packet format

| Format | Length (Byte) | illustrate |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 (packet length 1 bit) or 0x79 0x79 (packet length 2 bits) |
| Packet length | 1(2) | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | Represents the type of transmission packet (see the table below for details) |
| Information content | N | According to different applications, the corresponding "protocol number" determines the specific content |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

# Detailed explanation of protocol number

| Login package ( with terminal type ) | 0x01 |
| --- | --- |
| GPS positioning package (UTC) | 0x22 |
| Heartbeat packet | 0x13 |
| Terminal online command reply ( device reply ) | 0x21 |
| Alarm data (UTC) | 0x26 |
| LBS alarm package ( requires platform support ) | 0x19 |
| Online instructions ( issued by the server ) | 0x80 |
| Time calibration package ( requires platform support ) | 0x8A |
| General package for information transmission | 0x94 |
| LBS multi-base station extended information package ( requires platform support ) | 0X28 |

## Protocol Packet Decomposition

  - Login Packet

Notes on the login package:

    - The login package is the information package that establishes a connection between the terminal and the platform, and sends the terminal product information to the platform

    - The GPRS connection is established successfully and a login packet is sent to the server. If a return packet is received within 5 seconds, the connection is considered normal. If not, the login packet will continue to be sent.

    - If no return packet is received from the server for more than 5 seconds, the login packet response is considered timed out.

    - After 3 times of timeout, the terminal starts the scheduled restart function

- Login package [ 0X01 ]

Login Packet

|  |  | lengt h | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x01 |
| Informatio n content | Terminal ID | 8 | For example: If the IMEI number is 123456789123456, the terminal ID is: 0x01 0x23 0x45 0x67 0x89 0x12 0x34 0x56 |
|  | Type identificatio n code | 2 | Determine the terminal type based on this identification code |
|  | Time zone language | 2 | See the table below for details |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Example data: 78 78 11 01 07 52 53 36 78 90 02 42 70 00 32 01 00 05 12 79 0D 0A

For example:

Original data: 78 78 11 01 07 52 53 36 78 90 02 42 70 00 32 01 00 05 12 79 0D 0A

Data packet type: Login data packet (01) Packet length: 17 (11)

IMEI number: 752533678900242 (0752533678900242)

Device model: GT700 (7000)

Time zone: East 8.0, Language: English (3201) Message sequence number: 5 (0005)

| One half byte bit15-bit 4 | 15 | The time zone is expanded by 100. |  |
| --- | --- | --- | --- |
|  | 14 |  |  |
|  | 13 |  |  |
|  | 12 |  |  |
|  | 11 |  |  |
|  | 10 |  |  |
|  | 9 |  |  |
|  | 8 |  |  |
|  | 7 |  |  |
|  | 6 |  |  |
|  | 5 |  |  |
|  | 4 |  |  |
| Lower half byte bit4-bit0 | 3 | Time Zone East and West |  |
|  | 2 | No definition yet |  |
|  | 1 | Language selection bit | 1 Chinese |
|  | 0 | Language selection bit | 0 - English |

Verification information: 1279 (1279) Time zone language

B it3 0-------Eastern Time Zone 1-------Western Time Zone

If: extension bit: 0X32 0X00 means East 8th District, GMT+8:00. Calculation method: 8*100=800, converted to hexadecimal, 0X0320

Extension bits: 0X4D 0XD8 indicates the 12th and 3/4 time zones, GMT-12:45. Calculation method: 12.45*100=124 5 , converted to hexadecimal, 0X04, 0XDD.

The algorithm here is to shift the calculated time zone value left four bits and then combine the time zone east, west, and language selection bits to save four bytes.

For example:

Original data: 78 78 05 01 00 05 9F F8 0D 0A Data packet type: Login packet server reply (01) Packet length: 5 (05)

Message sequence number: 5 (0005)

Checksum information: 9FF8 (9FF8)

- Login packet reply (server reply)

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x01 |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

## Heartbeat packet [0X13]

Notes on heartbeat packets:

    - In order to keep the GPRS link from being disconnected, the link is sent to maintain the link;

    - The GPRS connection is established successfully and a heartbeat packet is sent to the server. If a return packet is received within 5 seconds, the connection is considered normal and the next heartbeat packet will be sent after the cycle.

    - If no return packet is received from the server for more than 5 seconds, the heartbeat is considered to have timed out.

    - After the heartbeat times out 3 times, the terminal starts the scheduled restart function

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x13 |
| Information content | Terminal information content | 1 | See the table below for details |
|  | Voltage level | 1 | 0x00: No power (power off) 0x01: The battery is extremely low (not enough to make calls or send text messages, etc.) 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) 0x04: Battery level 0x05: High battery 0x06: Battery level is extremely high |
|  | GSM signal strength | 1 | 0x00: no signal; 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal 0x04: Strong signal |
|  | Language/Extension port status | 2 | The last bit is 0x01 Chinese and 0x02 English |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

The terminal sends a heartbeat packet Heartbeat packet

For example:

Original data: 78 78 0A 13 40 04 04 00 01 00 0F DC EE 0D 0A

Data packet type: Heartbeat packet (13) Packet length: 10 (0A)

Hybrid: Connected, GPS: Positioned, External Power Status: Not Charging, ACC Status: Low, Anti-theft Status: Disarmed (40)

Voltage level: Medium power (04) GSM signal strength: Strong signal (04) Language: Chinese (0001)

Message sequence number: 15 (000F) Verification information: DCEE (DCEE)

Detailed explanation of terminal information content

Occupies 1 byte and is used to represent various status information of the mobile phone.

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric connected |
|  | Bit6 | 1: GPS positioning |
|  |  | 0: GPS not positioned |
|  | Bit3～Bit5 | Extension bit |
|  | Bit2 | 1: Connected to power and charging |
|  |  | 0: Not connected to a power source for charging |
|  | Bit1 | 1: ACC high |
|  |  | 0: ACC low |
|  | Bit0 | 1: Fortification |
|  |  | 0: Disarm |

- Server heartbeat packet reply For example:

Original data: 78 78 05 13 01 00 E1 A0 0D 0A

Data packet type: Heartbeat packet server reply (13) Packet length: 5 (05)

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x13 |

Message sequence number: 256 (0100) Verification information: E1A0 (E1A0)

|  | Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| --- | --- | --- | --- |
|  | Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

## GPS positioning package [0X22]

Description of GPS positioning package:

    - Data packets used to transmit the terminal's location

    - After positioning and connection are successful, upload positioning data points according to the set rules

    - If the connection is successful and there is a cached positioning point, the previously located data point will be retransmitted.

|  |  | length | Detailed explanation |  |  |
| --- | --- | --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |  |  |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |  |  |
| Protocol Number |  | 1 |  | 0x22 (UTC) |  |
| Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |  |  |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |  |  |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |  |  |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |  |  |
|  | speed | 1 | Convert to decimal to get the result |  |  |
|  | Heading, status | 2 | Convert to a 16-bit binary number and perform calculations based on the bits (see the table below for details) |  |  |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |  |  |
|  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |  |  |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |  |  |
|  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |  |  |
|  | ACC | 1 | ACC status ACC low is 00, ACC high is 01 (06 does not have this function) |  |  |
|  | Data reporting mode | 1 | GPS data point reporting type (06 does not have this function) 0x00 Regular reporting 0x01 Fixed-distance reporting 0x02 Inflection point upload 0x03 ACC status change upload 0x04 After changing from motion to stillness, retransmit the last positioning point 0x05 After the network is disconnected and reconnected, the last valid upload point is reported 0X06 reporting mode: ephemeris update forced upload of GPS points 0X07 reporting mode: press the button to upload the positioning |  |  |

The terminal sends a positioning packet Positioning package

|  |  |  |  | point 0X08 reporting mode: reporting location information when powered on 0X09 reporting mode: not used 0X0A reporting mode: the device reports the last latitude and longitude after being stationary, but the time is updated 0X0B WIFI parsing latitude and longitude upload package 0X0C reporting mode: LJDW (immediate positioning) command reporting 0X0D reporting mode: the device reports the last latitude and longitude after being stationary 0X0E reporting mode: GPSDUP upload (timing upload in static state) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GPS real-time transmission | 1 | 0x00 Real-time upload 0x01 Supplementary upload |  |  |  |
|  |  | Mileage statistics | 4 | Convert to decimal to get the result (some products have this function, some do not have this function and there is no such position) |  |  |  |
|  |  | Fuel sensor data | N |  | Fill in the information here according to the fuel sensor format; |  |  |
|  |  |  |  |  | if there is no information, fill in 0. |  |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |  |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |  |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |  |  |

For example:

Original data: 78 78 22 22 0F 0C 1D 02 33 05 C9 02 7A C8 18 0C 46 58 60 00 14 00 01 CC 00 28

7D 00 1F 71 00 00 01 00 08 20 86 0D 0A

Data packet type: 2G_GPS positioning data packet (22) Packet length: 34(22)

Time: 15-12-29 02:51:05(0F0C1D023305)

The GPS message length is 12, and the number of satellites involved in positioning is 9 (C9) Latitude = 23.111693333333335° North (027AC818)

Longitude = 114.40929777777778° East (0C465860)

Map link: https://www.google.com/maps?q=23.111693333333335,114.40929777777778 Speed = 0 km/h (00)

Positioning mode: Real-time GPS, Positioning status: GPS positioned, Heading: 0° (1400) MCC: 460 (01CC), MNC: 0 (00), LAC: 10365 (287D), Cell ID: 8049 (001F71)

ACC low (00), timing report (00), data retransmission (01)

Message sequence number: 8 (0008)

Verification information: 2086 (2086)

i.	Detailed explanation of status heading

| BYTE_1 | Bit7 | 0 |
| --- | --- | --- |
|  | Bit6 | 0 |
|  | Bit5 | GPS real-time/differ ential positioning |
|  | Bit4 | GPS positioning has been |
|  | Bit3 | East longitude, West longitude |
|  | Bit2 | South latitude, north latitude |
|  | Bit1 | course |
|  | Bit0 |  |
| BYTE_2 | Bit7 |  |
|  | Bit6 |  |
|  | Bit5 |  |
|  | Bit4 |  |
|  | Bit3 |  |
|  | Bit2 |  |
|  | Bit1 |  |
|  | Bit0 |  |

It occupies 2 bytes and indicates the direction of GPS operation. The range is 0 to 360 degrees. The unit is degree. The north is 0 degrees and the direction is clockwise.

- Server Location Packet Reply

The positioning packet server does not need to reply

## LBS multi-base station extended information package [0X28]

of LBS multi-base station extended information package :

    - Data packets used to transmit the location of the terminal when it is not positioned

- The terminal sends LBS multi-base station package

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x28 |
| Information content | Date and time (UTC) | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  | CI | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | RSSI | 1 | Cell signal strength, the value range is 0x00 ~ 0xFF, 0x00 is the weakest signal, 0xFF is the strongest signal. |
|  | NLAC1 | 2 | Same as above LAC |
|  | NCI1 | 3 | Same as above CI |
|  | NRSSI1 | 1 | Same as RSSI |
|  | NLAC2 | 2 | Same as above LAC |
|  | NCI2 | 3 | Same as above CI |
|  | NRSSI2 | 1 | Same as RSSI |
|  | NLAC3 | 2 | Same as above LAC |
|  | NCI3 | 3 | Same as above CI |
|  | NRSSI3 | 1 | Same as RSSI |
|  | NLAC4 | 2 | Same as above LAC |
|  | NCI4 | 3 | Same as above CI |
|  | NRSSI4 | 1 | Same as RSSI |
|  | NLAC5 | 2 | Same as above LAC |
|  | NCI5 | 3 | Same as above CI |
|  | NRSSI5 | 1 | Same as above RSSI |
|  | NLAC6 | 2 | Same as above LAC |
|  | NCI6 | 3 | Same as above CI |
|  | NRSSI6 | 1 | Same as RSSI |
|  | Time advance | 1 | It refers to the difference between the actual time it takes for a mobile station signal to reach the base |

|  |  |  |  | station and the time it takes for the mobile station signal to reach the base station assuming the distance between the mobile station and the base station is 0. |  |
| --- | --- | --- | --- | --- | --- |
|  |  | language | 2 | The last bit is 0x01 Chinese and 0x02 English |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

For example:

Original data: 78 78 3B 28 10 01 0D 02 02 02 01 CC 00 28 7D 00 1F 71 3E 28 7D 00

### 1F 72 31 28 7D 00 1E 23 2D 28 7D 00 1F 40 18 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 FF 00 02 00 05 B1 4B 0D 0A

Packet Type: 2G_LBS Multi-Base Station Extended Information Packet (28) Packet length: 59 (3B)

#### Time: 2013-01-16 02:02:02 (10010D020202) MCC: 460(01CC), MNC: 0(00), LAC: 10365(287D), Cell ID: 8049(001F71) RSSI: 62(3E)

LAC1: 10365(287D), Cell ID1: 8050(001F72), RSSI1: 49(31) LAC2: 10365(287D), Cell ID2: 7715(001E23), RSSI2: 45(2D) LAC3: 10365(287D), Cell ID3: 8000(001F40), RSSI3: 24(18) LAC4: 0(0000), Cell ID4: 0(000000), RSSI4: 0(00)

LAC5: 0(0000), Cell ID5: 0(000000), RSSI5: 0(00)

LAC6: 0(0000), Cell ID6: 0(000000), RSSI6: 0(00)

#### Timing advance: 255 (FF) Language: English (0002)

Message sequence number: 5 (0005) Verification information: B14B (B14B)

- Server LBS multi-base station package reply

LBS multi-base station package server does not need to reply

## Alarm package [0X26]

of the alarm package:

    - Used to transmit terminal-defined alarm content

    - After receiving the alarm content, the server responds and resolves the longitude and latitude into an address and sends it back to the terminal

    - The terminal sends the returned address to the SOS number set on the terminal

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x26 (UTC) |
| Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | speed | 1 | Convert to decimal to get the result |
|  | Heading, status | 2 | Convert to 16-bit binary number and calculate based on the bit (same as GPS package, see GPS package explanation for details) |
|  | LBS length | 1 | Total length of LBS information (length + MCC + MNC + CellID) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Terminal Information | 1 | See the table below for details |
|  | Voltage level | 1 | 0x00: No power (power off) 0x01: The battery is extremely low (not enough to make calls or send text messages, etc.) 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) 0x04: Battery level 0x05: High battery 0x06: Extremely high battery |
|  | GSM signal level | 1 | 0x00: no signal; 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal |

The terminal sends an alarm packet Alarm package (single fence)

|  |  |  |  | 0x04: Strong signal |
| --- | --- | --- | --- | --- |
|  |  | Alarm, language | 2 | See the table below for details |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

Original data: 78 78 25 26 0F 0C 1D 03 0B 26 C9 02 7A C8 18 0C 46 58 60 00 04 00 09 01 CC 00 28

7D 00 1F 71 80 04 04 13 02 00 0C 47 2A 0D 0A

Data packet type: 2G alarm data packet (26) Packet length: 37 (25)

Time: 15-12-29 03:11:38 (0F0C1D030B26)

The GPS message length is 12, and the number of satellites involved in positioning is 9 (C9) Latitude = 23.111693333333335° North (027AC818)

Longitude = 114.40929777777778° East (0C465860)

Map link: https://www.google.com/maps?q=23.111693333333335,114.40929777777778 Speed = 0 km/h (00)

Positioning mode: Real-time GPS, Positioning status: GPS not positioned, Heading: 0° (0400)

LBS Length: 9 (09)

MCC: 460 (01CC), MNC: 0 (00), LAC: 10365 (287D), Cell ID: 8049 (001F71)

Hybrid: Disconnected, GPS: Not Positioned, External Power Status: Not Charging, ACC Status: Low, Anti-Theft Status: Released (80)

Voltage level: Medium power (04) GSM signal strength: Strong signal (04) Alarm Status: Disassembly Alarm (13) Language: English(02)

Message sequence number: 12 (000C) Verification Information: 472A (472A)

Alarm package (multi-fence)

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x27 (UTC) |

|  |  | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
| --- | --- | --- | --- | --- |
|  |  | Number of GPS |  | The first character is the length of the GPS information, and the |
|  |  | information | 1 | second character is the number of satellites involved in positioning |
|  |  | satellites |  | (converted to decimal) |
|  |  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  |  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  |  | speed | 1 | Convert to decimal to get the result |
|  |  | Heading, status | 2 | Convert to 16-bit binary number and perform calculation based on the bit (same as GPS package, see GPS package explanation for details) |
|  |  | LBS length | 1 | Total length of LBS information (length + MCC + MNC + CellID) |
|  |  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  |  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  |  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  |  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Information | Terminal Information | 1 | See the table below for details |
|  | content |  |  | 0x00: No power (power off) |
|  |  |  |  | 0x01: The battery is extremely low (not enough to make calls or send |
|  |  |  |  | text messages, etc.) |
|  |  | Voltage level | 1 | 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) |
|  |  |  |  | 0x04: Battery level |
|  |  |  |  | 0x05: High battery |
|  |  |  |  | 0x06: Battery level is extremely high |
|  |  |  |  | 0x00: no signal; |
|  |  | GSM signal level | 1 | 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal |
|  |  |  |  | 0x04: Strong signal |
|  |  | Alarm, language | 2 | See the table below for details |
|  |  | Fence number | 1 | Fence alarm: This bit is valid, 1 is fence 1, 2 is fence 2 , and FF is invalid. |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  |  |  |  | The CRC-ITU value from "Packet Length" to "Message Sequence Number". |
|  | Error checking |  | 2 | If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix |
|  |  |  |  | 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 26 27 10 04 19 09 2D 07 C5 02 7A C9 1C 0C 46 58 00 00 05 37 09 00 00 00 00

00 00 00 00 80 02 00 0C 01 FF 00 00 4D F6 0D 0A

  - Detailed explanation of terminal information

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric connected |
|  | Bit6 | 1: GPS positioning |
|  |  | 0: GPS not positioned |
|  | Bit3～Bit5 | 100: SOS |
|  |  | 011: Low battery alarm |
|  |  | 010: Power failure alarm |
|  |  | 001: Vibration alarm |
|  |  | 000: Normal |
|  | Bit2 | 1: Connected to power and charging |
|  |  | 0: Not connected to a power source for charging |
|  | Bit1 | 1: ACC high |
|  |  | 0: ACC low |
|  | Bit0 | 1: Fortification |
|  |  | 0: Disarm |

  - Detailed explanation of alarm language [support is currently supported by the device]

| Byte 1 | 0x00: Normal [support] |
| --- | --- |
|  | 0x01: SOS [support] |
|  | 0x02: Power failure alarm [support] |
|  | 0x03: Vibration alarm [support] |
|  | 0x04: Fence entry alarm [support] |
|  | 0x05: Fence out alarm [support] |
|  | 0x06 Overspeed alarm [support] |
|  | 0x09 Displacement alarm [support] |
|  | 0x0A Entering GPS blind area alarm [not supported] |
|  | 0x0B GPS blind zone alarm [not supported] |
|  | 0x0C Power-on alarm [not supported] |
|  | 0x0D GPS first positioning alarm [not |

|  |  | supported] |  |
| --- | --- | --- | --- |
|  |  | 0x0E External low power alarm [support] |  |
|  |  | 0x0F External low power protection alarm [support] |  |
|  |  | 0x10 Card replacement alarm [optional] |  |
|  |  | 0x11 Shutdown alarm [not supported] |  |
|  |  | 0x12 Flight mode alarm after external low battery protection [not supported] |  |
|  |  | 0x13 Light sensor removal alarm [support] |  |
|  |  | 0x14 Door alarm [support] |  |
|  |  | 0x15 Low power shutdown alarm [not supported] |  |
|  |  | 0x16 Voice-activated alarm [not supported] |  |
|  |  | 0x17 Fake base station alarm [support] |  |
|  |  | 0x18 Cover open alarm [not supported] |  |
|  |  | 0x19 Internal battery low power alarm [support] |  |
|  |  | 0x1A Exit transport mode alarm [not supported] |  |
|  |  | 0x1B Suspected Liqun alarm [not supported] |  |
|  |  | 0x20 Deep sleep alarm (flight) [not supported] |  |
|  |  | 0x21 GPS interference alarm [support] |  |
|  |  | 0x22 Reserved |  |
|  |  | 0x23 Fall alarm [not supported] |  |
|  |  | 0x24 power restore [supported] |  |
|  |  | 0x25 Disassembly alarm [not supported] |  |
|  |  | 0x26 Stay away from Bluetooth alarm [not supported] |  |
|  |  | 0x27 Thread cutting alarm [not supported] |  |
|  |  | 0x28 Active offline (shutdown) alarm [not supported] |  |
|  |  | 0x29 Rapid acceleration [support] |  |
|  |  | 0x2A Left sharp turn alarm [support] |  |
|  |  | 0x2B Right sharp turn alarm [support] |  |
|  |  | 0x2C Collision alarm [support] |  |
|  |  | 0x2D Vehicle rollover alarm [support] |  |
|  |  | 0x30 Rapid deceleration [support] |  |
|  |  | 0x31 Out-of-group alarm [not supported] |  |
|  |  | 0x32 Remove flip alarm [ support] |  |
|  |  | 0x33 Lock alarm [not supported] |  |
|  |  | 0x34 Unlock alarm [not supported] |  |

|  |  | 0x35 Abnormal unlocking alarm [not supported] |  |
| --- | --- | --- | --- |
|  |  | 0x36 Unlock failure alarm [not supported] |  |
|  |  | 0x37 Knock alarm [not supported] |  |
|  |  | 0x38 Over-distance alarm [not supported] |  |
|  |  | 0x39 Over-range alarm cancellation [not supported] |  |
|  |  | 0x3A Wristband recovery [not supported] |  |
|  |  | 0x3B Inactivity alarm [not supported] |  |
|  |  | 0x3C Car theft alarm [not supported] |  |
|  |  | 0x3D Illegal startup alarm [support] |  |
|  |  | 0x3E Key event reporting [not supported] |  |
|  |  | 0x3 F Tilt alarm [support] |  |
|  |  | 0x47 Fatigue Driving [support] |  |
|  |  | 0x 48 Pet lost alarm [not supported] |  |
|  |  | 0x 49 Battery full alarm [not supported] |  |
|  |  | 0x4A Battery abnormality alarm [not supported] |  |
|  |  | 0x4 B External GNSS alarm [support] |  |
|  |  | 0X4C Air conditioning on alarm (optional) |  |
|  |  | 0X4D Air conditioning off alarm (optional) |  |
|  |  | 0x4E Vehicle stability (optional) |  |
|  |  | 0x4F Vehicle free fall [optional] |  |
|  |  | 0xFF ACC off |  |
|  |  | 0xFE ACC open |  |
|  | Byte 2 | 0x01 Chinese 0x02 English 0x00 No platform response required |  |

- Server alarm packet reply

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x26 (UTC) |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix |

|  |  |  | 1 for the algorithm). |
| --- | --- | --- | --- |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

Original data: 78 78 05 26 00 1C 9D 86 0D 0A

Data packet type: 2G alarm data packet response packet (26) Packet length: 5 (05)

Message sequence number: 28 (001C) Verification information: 9D86 (9D86)

## Alarm package (LBS) [0X19] (This package is not available for vehicle-mounted and vehicle-mounted OBD )

of the alarm package:

    - Used to transmit terminal-defined alarm content

    - After receiving the alarm content, the server responds and resolves the LBS base station information into an address and sends it back to the terminal.

    - The terminal sends the returned address to the SOS number set on the terminal

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x19 |
| Informati on content | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) (converted to decimal) |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Terminal Information | 1 | See the table below for details |
|  | Voltage level | 1 | 0x00: No power (power off) 0x01: The battery is extremely low (not enough to make calls or send text messages, etc.) 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) 0x04: Battery level 0x05: High battery 0x06: Battery level is extremely high |
|  | GSM signal level | 1 | 0x00: no signal; 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal 0x04: Strong signal |
|  | Alarm, language | 2 | See the table below for details |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |

The terminal sends an alarm packet Alarm package

Fixed value, unified as 0x0D 0x0A

2

Stop bits

For example:

Original data: 78 78 12 19 01 CC 00 28 7D 00 1F 71 20 04 04 01 01 00 94 6C 89 0D 0A

Data  packet  type:  2G_LBS  alarm  data  packet  (19) Packet length: 18 (12)

MCC: 460 (01CC), MNC: 0 (00), LAC: 10365 (287D), Cell ID: 8049 (001F71)

Hybrid:Connected,GPS:NotPositioned,ExternalPowerStatus:NotCharging,ACCStatus:Low,                      Anti-theft Status: Disarmed (20)

Voltage level: Medium power (04)

GSM  signal  strength:  Strong  signal  (04) Alarm status: SOS alarm (01)

Language: Chinese (01)

Message  Sequence  Number:  148  (0094) Verification information: 6C89 (6C89)

Note:  To  be  compatible  with  some  countries  where  MNC  occupies  two  bytes  ,  the  highest  bit  of MCC is used to distinguish the MNC length .

When1,theMNClengthis2.ThedefaultvalueofBit15oftheolddeviceshippedis0;thedefault                     value of Bit 15 of the new device is 1.

MCC position detailed explanation

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE S | Bit15 | 1: MNC length is 2 |
|  |  | 0: MNC length is 1 |
|  | Bit0~bit14 | MCC Information |

  - Detailed explanation of terminal information

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric connected |
|  | Bit6 | 1: GPS positioning |
|  |  | 0: GPS not positioned |
|  | Bit3～Bit5 | 100: SOS |
|  |  | 011: Low battery alarm |
|  |  | 010: Power failure alarm |
|  |  | 001: Vibration alarm |

|  |  |  | 000: Normal |  |
| --- | --- | --- | --- | --- |
|  |  | Bit2 | 1: Connected to power and charging |  |
|  |  |  | 0: Not connected to a power source for charging |  |
|  |  | Bit1 | 1: ACC high |  |
|  |  |  | 0: ACC low |  |
|  |  | Bit0 | 1: Fortification |  |
|  |  |  | 0: Disarm |  |

  - Detailed explanation of alarm language [support is currently supported by the device]

| Byte 1 | 0x00: Normal [support] |
| --- | --- |
|  | 0x01: SOS [support] |
|  | 0x02: Power failure alarm [support] |
|  | 0x03: Vibration alarm [support] |
|  | 0x04: Fence entry alarm [support] |
|  | 0x05: Fence out alarm [support] |
|  | 0x06 Overspeed alarm [support] |
|  | 0x09 Displacement alarm [support] |
|  | 0x0A Entering GPS blind area alarm [not supported] |
|  | 0x0B GPS blind zone alarm [not supported] |
|  | 0x0C Power-on alarm [not supported] |
|  | 0x0D GPS first positioning alarm [not supported] |
|  | 0x0E External low power alarm [support] |
|  | 0x0F External low power protection alarm [support] |
|  | 0x10 Card replacement alarm [optional] |
|  | 0x11 Shutdown alarm [not supported] |
|  | 0x12 Flight mode alarm after external low battery protection [not supported] |
|  | 0x13 Light sensor removal alarm [support] |
|  | 0x14 Door alarm [support] |
|  | 0x15 Low power shutdown alarm [not supported] |
|  | 0x16 Voice-activated alarm [not supported] |
|  | 0x17 Fake base station alarm [support] |
|  | 0x18 Cover open alarm [not supported] |
|  | 0x19 Internal battery low power alarm [support] |

|  |  | 0x1A Exit transport mode alarm [not supported] |  |
| --- | --- | --- | --- |
|  |  | 0x1B Suspected Liqun alarm [not supported] |  |
|  |  | 0x20 Deep sleep alarm (flight) [not supported] |  |
|  |  | 0x21 GPS interference alarm [support] |  |
|  |  | 0x22 Reserved |  |
|  |  | 0x23 Fall alarm [not supported] |  |
|  |  | 0x24 Alarm when charger is plugged in [not supported] |  |
|  |  | 0x25 Disassembly alarm [not supported] |  |
|  |  | 0x26 Stay away from Bluetooth alarm [not supported] |  |
|  |  | 0x27 Thread cutting alarm [not supported] |  |
|  |  | 0x28 Active offline (shutdown) alarm [not supported] |  |
|  |  | 0x29 Rapid acceleration [support] |  |
|  |  | 0x2A Left sharp turn alarm [support] |  |
|  |  | 0x2B Right sharp turn alarm [support] |  |
|  |  | 0x2C Collision alarm [support] |  |
|  |  | 0x2D Vehicle rollover alarm [support] |  |
|  |  | 0x30 Rapid deceleration [support] |  |
|  |  | 0x31 Out-of-group alarm [not supported] |  |
|  |  | 0x32 Remove flip alarm [ support] |  |
|  |  | 0x33 Lock alarm [not supported] |  |
|  |  | 0x34 Unlock alarm [not supported] |  |
|  |  | 0x35 Abnormal unlocking alarm [not supported] |  |
|  |  | 0x36 Unlock failure alarm [not supported] |  |
|  |  | 0x37 Knock alarm [not supported] |  |
|  |  | 0x38 Over-distance alarm [not supported] |  |
|  |  | 0x39 Over-range alarm cancellation [not supported] |  |
|  |  | 0x3A Wristband recovery [not supported] |  |
|  |  | 0x3B Inactivity alarm [not supported] |  |
|  |  | 0x3C Car theft alarm [not supported] |  |
|  |  | 0x3D Illegal startup alarm [support] |  |
|  |  | 0x3E Key event reporting [not supported] |  |
|  |  | 0x3 F Tilt alarm [support] |  |
|  |  | 0x47 Fatigue Driving [support] |  |
|  |  | 0x 48 Pet lost alarm [not supported] |  |

|  |  | 0x 49 Battery full alarm [not supported] |  |
| --- | --- | --- | --- |
|  |  | 0x4A Battery abnormality alarm [not supported] |  |
|  |  | 0x4 B External GNSS alarm [support] |  |
|  |  | 0X4C Air conditioning on alarm (optional) |  |
|  |  | 0X4D Air conditioning off alarm (optional) |  |
|  |  | 0x4E Vehicle stability (optional) |  |
|  |  | 0x4F Vehicle free fall [optional] |  |
|  |  | 0xFF ACC off |  |
|  |  | 0xFE ACC open |  |
|  | Byte 2 | 0x01 Chinese 0x02 English 0x00 No platform response required |  |

- Server alarm packet reply

|  | lengt h | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x26 (UTC) |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

Original data: 78 78 05 26 00 1C 9D 86 0D 0A

Data packet type: 2G alarm data packet response packet (26) Packet length: 5 (05)

Message sequence number: 28 (001C) Verification information: 9D86 (9D86)

## Online instruction [0X80]

Description of the online instruction package:

    - Used by the server to send online instructions to control the terminal to perform corresponding tasks

    - After receiving the result, the terminal replies to the server

- Server online command sending

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x80 |
| Information content | Instruction length | 1 | Server flag + command content length |
|  | Server flags | 4 | The server will use it to identify the data. The terminal will return the received data in binary format in the return packet. |
|  | Instruction content | M | Indicated in ASCII string, the command content is compatible with SMS commands |
|  | language | 2 | The last bit is 0x01 Chinese and 0x02 English |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

Original data: 78 78 0E 80 08 00 00 00 00 73 6F 73 23 00 01 6D 6A 0D 0A

Data packet type: Online command packet (80) Packet length: 14 (0E)

Command length: 8 (08)

Server Flags: 00000000 (00000000) Command content: sos# (736F7323) Language: Missing language bit Message sequence number: 1 (0001) Verification information: 6D6A (6D6A)

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x7 9 0x7 9 |
| Packet length | 2 | Length = protocol number + information content + information |

Terminal online command reply Terminal reply (general command)

|  |  |  |  | sequence number + error check |
| --- | --- | --- | --- | --- |
|  | Protocol Number |  | 1 | 0x21 |
|  | Information content | Server flags | 4 | The server will use it to identify the data. The terminal will return the received data in binary format in the return packet. |
|  |  | Content Encoding | 1 | 0x01 ASCII encoding 0x02 UTF16-BE encoding |
|  |  | content | M | The data to be sent (according to the content encoding format) |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

3A 53 74 72 6F 6E 67 3B 20 47 50 53 3A 53 65 61 72 63 68 69 6E 67 20 73 61 74 65 6C 6C 69 74

65 2C 20 53 56 53 20 55 73 65 64 20 69 6E 20 66 69 78 3A 30 28 30 29 2C 20 47 50 53 20 53 69

67 6E 61 6C 20 4C 65 76 65 6C 3A 3B 20 41 43 43 3A 4F 46 46 3B 20 44 65 66 65 6E 73 65 3A 4F

46 46 00 2E 26DF 0D 0A

Data packet type: Online command reply packet (21) Packet length: 157 (009D)

Server Flags: 00000000 (00000000) Encoding type: ASCII encoding (01) Command reply content:

Battery: 4.16V, normal; GPRS: connected normally; GSM signal strength: strong; GPS: searching for satellites, SVS positioning in use: 0(0), GPS signal strength:; ACC: off; Defense: off(426174746572793A342E3136562C4E4F524D414C3B20475052533A4C696E6B2055703B20 47534D205369676E616C204C6576656C3A5374726F6E673B204750533A536561726368696 E6720736174656C6C6974652C20535653205573656420696E206669783A302830292C204750 53205369676E616C204C6576656C3A3B204143433A4F46463B20446566656E73653A4F4646)

Message sequence number: 46 (002E) Checksum information: 26DF (26DF)

## Time calibration package [0X8A]

Description of the time calibration package:

    - Used to automatically request time synchronization from the server when the terminal is turned on, to solve the problem of time error when the terminal is not positioned when it is turned on

    - The server responds with the correct time and format, the time is UTC time

- The terminal sends a time calibration request

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x8A |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

For example:

Original data: 78 78 05 8A 00 06 88 29 0D 0A Packet type: Network time request packet (8A) Packet length: 5 (05)

Message sequence number: 6 (0006)

Verification information: 8829 (8829)

- The server responds with time information

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x8A (UTC) |
| Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will |

|  |  |  | be discarded (see Appendix 1 for the algorithm). |  |
| --- | --- | --- | --- | --- |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |  |

For example:

Original data: 78 78 0B 8A 0F 0C 1D 00 00 15 00 06 F0 86 0D 0A

Packet Type: Time Request Response (8A) Packet length: 11 (0B)

Time: 15-12-29 00:00:21 (0F0C1D000015)

Message sequence number: 6 (0006) Verification Information: F086 (F086)

#### Information transmission general package [0X94]

Description of the general package for information transmission:

    - Used for terminal transmission of various non-positioning data

- Terminal sends general information transmission package

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x94 |
| Information content | Message type (subprotocol number) | 1 | 00 External voltage 01~03 (customized) 04 Terminal Status Synchronization 05 Door Status 0x0D: Oil sensor reports data ...To be added |
|  | Data content | N | The transmission content varies depending on the information type. See the table below for details. |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Example data: 79 79 00 7F 94 04 41 4C 4D 31 3D 43 34 3B 41 4C 4D 32 3D 43 43 3B 41 4C 4D 33

3D 34 43 3B 53 54 41 31 3D 43 30 3B 44 59 44 3D 30 31 3B 53 4F 53 3D 2C 2C 3B 43 45 4E 54 45

52 3D 3B 46 45 4E 43 45 3D 46 65 6E 63 65 2C 4F 4E 2C 30 2C 32 33 2E 31 31 31 38 30 39 2C 31

31 34 2E 34 30 39 32 36 34 2C 34 30 30 2C 49 4E 20 6F 72 20 4F 55 54 2C 30 3B 4D 49 46 49 3D

4D 49 46 49 2C 4F 46 46 00 0A 06 1E 0D 0A

Transmitted information content

# When the type is 00

This bit transmits the external voltage. This is a two-digit hexadecimal number. After converting the hexadecimal number to decimal, divide it by 100. For example, 0X04, 0X9F, 049F is converted to decimal as 1183, which is divided by 100 to 11.83, indicating that the external voltage value of the terminal is 11.83V.

Original data: 79 79 00 08 94 00 05 7D 06 65 7B AA 0D 0A

Packet type: General packet for information transmission (94) Packet length: 8 (0008)

Information type: External voltage (00) External voltage: 14.05V (057D) Message sequence number: 1637 (0665) Checksum information: 7BAA (7BAA)

# When the type is 0 1 ( new customization )

This bit transmits the ADC analog value . This is a two-digit hexadecimal number. After converting the hexadecimal number to decimal, divide it by 100. For example, 0X04, 0X9F, 049F is converted to decimal as 1183, which is divided by 100 to 11.83, indicating that the terminal external voltage value is 11.83V.

# When type is 0 2 ( altitude customization )

Original data: $GNGGA,065601.000,2940.21222,N,10629.49452,E,1,29,0.6,356.9,M,-39.0,M,,*69

UTC time of current positioning: 06:56:01:000

Latitude: 29.670203666666666 (2940.21222), latitude direction: North (N)

Longitude: 106.49157533333333 (10629.49452), longitude direction: East (E) Current positioning quality: SPS positioning mode, positioning is valid (1) Number of satellites: 29

Horizontal precision factor: 0.6

Altitude: 356.9, unit: M

Distance between reference ellipsoid and geoid: -39.0, unit: M Data age of differential correction:

ID of differential reference station:

Checksum: 69 (69)

This bit transmits the poster parameter , which is a 2 -digit hexadecimal number . For example, the altitude is represented by 356 meters in decimal and 0X0164 in hexadecimal.

Example protocol:

Original data: 79 79 00 08 94 02 01 64 00 01 77 B7 0D 0A

Packet type: General packet for information transmission (94) Packet length: 8 (0008)

Information type: Altitude customization (02) Information content: 356 meters (0164)

Message sequence number: 1 (0001) Checksum information: 77B7 (77B7)

# 94 type is 03 ( Bluetooth oil level sensor -MIELTA FANTOM)

| Level LSB Oil sense low byte Level MSB Oil sense high byte Range: The sensor transmits values from 30 to 4095 by default | 2 | Example: 0X0A8 0 MSB=0A LSB=80 level is 2688 2688/4095=0.65=65% The value is transmitted in the advertising packet as the value 9C40. The byte order in the packet is 409C. If the value 9C40 is converted to decimal code, it is 40000. The value 40000 is sent by an uncalibrated fuel level sensor. |
| --- | --- | --- |
| Level, perсent Oiliness percentage Percentage level, 0..100 % | 1 | Example: 0X10 gasoline percentage 16% |
| VBat Oil sensor battery voltage Battery voltage, 100 мв/бит | 1 | 0X24=36 36/10=3.6V |
| Temperature temperature Temperature (int8_t), [-127..+127] | 2 | 01:10: 27.2 degrees Celsius FFFF: Invalid data, the temperature sensor is not connected or the sensor is damaged 8016: (the highest bit is 1 to indicate negative temperature) -2.2 degrees Celsius |
| Accel Z ->angle angle Angle, [0…180] | 1 | 0X05=5°, the range is 0-180°, no negative value |
| Flags 1 | 1 |  |

Deciphering the Flags 1 bit field

- bit 0: flag of stability of meter values - 0: frequency is unstable, 1: frequency is stable. Here, 5 consecutive

measurements are used for calculations.

- bit 1: flag of stability of the level in the tank - 0: the level is unstable, 1: the level is stable. The number of

Consecutive measurements for calculations are set by the smoothing parameter.

- bit 2: flag of the covering sensor: 0 - the sensor is not active, 1 - the sensor is active (triggered)

Interpretation of the flags 1 bit field

- Bit 0: Instrument value stability flag - 0: Frequency unstable, 1: Frequency stable. 5 consecutive measured values are used for calculation.

- Bit 1: Tank level stability flag - 0: level unstable, 1: level stable. The number of consecutive measurements used for the calculation is set by the smoothing parameter.

- Bit 2: Override sensor flag: 0 - sensor not activated, 1 - sensor

activated (triggered)

0X06=00000100: bit 0 is not set, bit 1 is not set, bit 2 is set; Note: Only the lower 3 bits are valid, the other bits are reserved for

the sensor and have no specific meaning.

Original data: 79 79 00 0E 94 03 06 EB 2A 24 01 40 03 62 06 68 3A DA 0D 0A

Packet type: General packet for information transmission (94) Packet length: 14 (000E)

Information type: Bluetooth fuel level sensor (03) Level sensor percentage: 43% (06EB)

Oiliness percentage: 42% (2A)

Oil sensor battery voltage: 3.6 (24)

Temperature: 32.0° (0140)

Sensor angle: 3° (03)

Instrument Value Stability: Frequency Unstable, Tank Level Stability: Level Stable, Cover Sensor Flag: Sensor Inactive (62)

Message sequence number: 1640 (0668) Calibration information: 3ADA (3ADA)

# When the type is 04

This bit transmits terminal status synchronization information. The length of this position is variable and the transmission

is ASCII code.

Content Identifier Definition

| definition | Identifier |
| --- | --- |
| Alarm bit 1 | ALM1 |
| Alarm bit 2 | ALM2 |
| Alarm bit 3 | ALM3 |
| Status bit 1 | STA1 |
| SOS number | SOS |
| Center Number | CENTER |
| Fence | FENCE |
| Oil and power off state | DYD |
| model | MODE |

  - ALM1 definition (status)

| Bit | definition | Remark |
| --- | --- | --- |
| bit7 | Vibration alarm | 1 on, 0 off |
| bit6 | Network alarm | 1 on, 0 off |
| bit5 | Call the police | 1 on, 0 off |
| bit4 | SMS alarm | 1 on, 0 off |
| bit3 | Displacement alarm | 1 on, 0 off |
| bit2 | Network alarm | 1 on, 0 off |
| bit1 | Call the police | 1 on, 0 off |
| bit0 | SMS alarm | 1 on, 0 off |

  - ALM2 Definition (Status)

| Bit | definition | Remark |
| --- | --- | --- |
| bit7 | Internal low battery alarm | 1 on, 0 off |
| bit6 | Network alarm | 1 on, 0 off |
| bit5 | Call the police | 1 on, 0 off |
| bit4 | SMS alarm | 1 on, 0 off |
| bit3 | External low battery alarm | 1 on, 0 off |
| bit2 | Network alarm | 1 on, 0 off |
| bit1 | Call the police | 1 on, 0 off |
| bit0 | SMS alarm | 1 on, 0 off |

  - ALM3 Definition (Status)

| Bit | definition | Remark |
| --- | --- | --- |

|  | bit7 | Overspeed alarm | 1 on, 0 off |  |
| --- | --- | --- | --- | --- |
|  | bit6 | Network alarm | 1 on, 0 off |  |
|  | bit5 | Call the police | 1 on, 0 off |  |
|  | bit4 | SMS alarm | 1 on, 0 off |  |
|  | bit3 | Power failure alarm | 1 on, 0 off |  |
|  | bit2 | Network alarm | 1 on, 0 off |  |
|  | bit1 | Call the police | 1 on, 0 off |  |
|  | bit0 | SMS alarm | 1 on, 0 off |  |

  - STA1 definition (status)

| Bit | definition | Remark |
| --- | --- | --- |
| bit7 | Armed state | 1 Arm 0 Disarm |
| bit6 | Automatic arming | 1 on, 0 off |
| bit5 | Manual arming | 1 on, 0 off |
| bit4 | Remote disarming | 1 on, 0 off |
| bit3 | To be defined |  |
| bit2 | To be defined |  |
| bit1 | Tamper switch closed | 1 closed 0 open |
| bit0 | Tamper alarm status | 1 on, 0 off |

- Definition of fuel and power failure status

| Bit | definition | Remark |  |  |
| --- | --- | --- | --- | --- |
| bit7 | Undefined |  |  |  |
| bit6 | Undefined |  |  |  |
| bit5 | Undefined |  |  |  |
| bit4 | Undefined |  |  |  |
| bit3 | Too high a speed and delayed execution | 1 This bit This bit is | is valid invalid | 0 |
| bit2 | Unlocated Deferred Execution | 1 This bit This bit is | is valid invalid | 0 |
| bit1 | Disconnect the oil and electricity | 1 This bit This bit is | is valid invalid | 0 |
| bit0 | Oil and electricity connection | 1 This bit This bit is | is valid invalid | 0 |

- SOS definition: ASCII transmission (multiple SOS numbers are separated by commas)

- Center number definition: ASCII transmission

- Fence definition: ASCII transmission

- Mode: ASCII transmission (parameters are separated by commas ",")

For example: ALM1=FF; ALM2=FF; ALM3=FF; STA1=CO ; DYD=01; SOS=12345,2345,5678; CENTER=987654; FENCE= FENCE,ON,0,-22.277120,-113.516763,5,IN,1; MODE= MODE,1,20,500

Note: This bit of content is not necessarily transmitted in its entirety. Please parse it according to the bit. The content reported by different products may vary depending on the product.

# When the type is 05

| Bit |  | definition | Remark |
| --- | --- | --- | --- |
| B | it1 5 | To be determined |  |
| B | it1 4 | To be determined |  |
| B | it1 3 | To be determined |  |
| B | it 12 | To be determined |  |
| B | it 11 | To be determined |  |
| Bit 10 |  | Input2 status | Air conditioning input customization/si de door detection, etc. 1Connect 0 Disconnect |
| B | it 9 | To be determined |  |
| Bit 8 |  | To be determined |  |
| bit7 |  | To be determined |  |
| bit6 |  | To be determined |  |

This bit transmits the terminal external IO port detection (side gate detection) status, and is transmitted as a hexadecimal number

|  | bit5 | To be determined |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bit4 | To be determined |  |  |  |  |
|  | bit3 | OUTPUT1/RELAY2 |  | 1 oil disconnect 0 oil connected |  |  |
|  | bit2 | IO port status | 1 High 0 Low |  |  |  |
|  | bit1 | Trigger status | 1 high trigger 0 low trigger |  |  |  |
|  | bit0 | Door status | 1 on, 0 off |  |  |  |

Original data: 79 79 00 08 94 05 00 00 03 69 F8 DA 0D 0A

Packet Type: General Information Transfer Packet (94) Packet length: 8 (0008)

Message Type: Door Status (05)

Door status: Closed, Trigger status: Low trigger, IO port status: Low, OUTPUT1/RELAY2 status: Connected, Digital input 2 (Input2): Close (0000)

Message sequence number: 873 (0369) Verification information: F8DA (F8DA)

# When the 94 type is 06 ( temperature and humidity integrated sensor )

5 or 9 bytes:

1 temperature occupies 2 bytes, 1 humidity occupies 2 bytes, each data occupies 2 bytes, unit 0.1 degree

The highest bit indicates the sign (1 is negative, 0 is positive) , for example, 0X8008 means minus 0.8 degrees Celsius

For example:

Example 1: 0110 1000: 0110 27.2 degrees Celsius 1000: Humidity integer 16% decimal .00% = 16%

Example 2: 8016 2006: 8016 (the highest bit is 1 to indicate negative temperature) -2.2 degrees Celsius 2006:

Humidity integer 32% decimal 0.06% = 32.6%

Example 3: FFFF FFFF: Second channel: invalid data, the temperature and humidity sensor is not connected or

the sensor is damaged

| 0x06 | T1 and T2 | H1 | H 1 | T1 and T2 | H 2 | H2 |
| --- | --- | --- | --- | --- | --- | --- |
| Sub- protocol | The highest | Humidit | Humidity | The highest | Humidity 2 | Humidity |
|  | bit of T1 | y 1 | 1 ( RH ) | bit of T1 | ( RH ) | 2 ( RH ) |
|  | 0x00: | ( RH ) | decimal | 0x00: | Integer | decimal |
|  | positive | Integer | point part | positive | part | point part |
|  | number | part |  | number |  |  |
|  | 0x80 : |  |  | 0x80 : |  |  |
|  | Negative |  |  | Negative |  |  |
|  | number |  |  | number |  |  |
|  | Temperature |  |  | Temperature |  |  |
|  | 1 |  |  | 1 |  |  |
|  | the integer |  |  | the integer |  |  |
|  | part from |  |  | part from |  |  |
|  | hexadecimal |  |  | hexadecimal |  |  |
|  | to decimal |  |  | to decimal |  |  |
|  | and divide |  |  | and divide by |  |  |
|  | by 10 to get |  |  | 10 to get the |  |  |
|  | the |  |  | temperature |  |  |
|  | temperature |  |  | value. |  |  |
|  | value. |  |  |  |  |  |

# When 94 type is 07 ( tire pressure sensor )

The device requests the tire pressure sensor to upload all tire data:

BLE sends: AA A5 5A 03 13 00 BF

M CU replied:

AA BB C0

01

15

15

AA 5A A5 0 4 13 00 02 BE The number of tires is 2, indicating that only 2 sensor data are matched.

### AA 5A A5 0B 13 00

AA BB C1

02

AA 5A A5 0B 13 00

or

#### 4B 20 80 ED 1st tire data 4B 20 00 6F 2nd tire data

| 15 | 4B | 20 | 80 |
| --- | --- | --- | --- |
| 15 | 4B | 20 | 00 |
| 15 | 4B | 20 | 80 |
| 15 | 4B | 20 | 00 |

AA 5A A5 0 4 13 00 0 4 C0	The number of tires is 2, indicating that only 2 sensor data are matched.

### AA 5A A5 0B 13 00 01 AA BB C2 AA 5A A5 0B 13 00 02 AA BB C3 AA 5A A5 0B 13 00 03 AA BB C4 AA 5A A5 0B 13 00 04 AA BB C5

#### F0 1st tire data 72 2nd tire data F4 3rd tire data 76 4th tire data

Protocol packet length: number of sensors * 8 + 1

| Number of sensors |  |  |  |  |  | 1 | Example data: 0X02 2 sensors installed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID1 Tire position |  |  |  |  |  | 1 | Sample Data 0x01 |  |
| Tire ID1 |  | data | 1 |  |  | 1 | Sample Data 0xAA |  |
| Tire ID 2 |  | data | 1 |  |  | 1 | Sample Data 0xBB |  |
| Tire ID 3 |  | data | 1 |  |  | 1 | Sample Data 0xC0 |  |
| Tire data 1 Tire pressure |  |  |  |  |  | 1 | Sample Data 0x 15 Pressure: The default pressure unit is Bar, hexadecimal to decimal, precision: 0.1 , for example: pressure = 0x15 , corresponding to pressure 2.1bar |  |
| Tire data 1 temperature |  |  |  |  |  | 1 | Sample Data 0x 4 B Temperature: The default temperature unit is ℃, hexadecimal to decimal, precision: 1, offset: -5 0 , for example: temperature = 0x 4 B, converted to decimal is 75 , then the sensor temperature is 75 -50 = 25 ℃ |  |
| Tire data 1 Sensor voltage |  |  |  |  |  | 1 | Sample Data 0X20 Voltage: sensor voltage value, hexadecimal to decimal, precision 0.1 , for example: voltage = 0x20 , the corresponding sensor voltage value is 3.2V |  |
| Tire data 1 state |  |  |  |  |  | 1 | Sample Data 0X80=10000000 Leakage, no high pressure, no low pressure, high temperature, no low battery |  |
|  | ....Sensor 2 Refer to |  |  |  |  |  |  |  |
|  | the package of sensor 1 |  |  |  |  |  |  |  |
|  | ....Sensor 3 Reference |  |  |  |  |  |  |  |
|  | Sensor 1 Package |  |  |  |  |  |  |  |
|  | ....Sensor 4 Reference |  |  |  |  |  |  |  |
|  | Sensor 1 Package |  |  |  |  |  |  |  |

# When the type is 0 D (oil sensor) (supported by BR1201)

- !AIOIL format

| Field | describe |
| --- | --- |
| Special beginning of the protocol | !AIOIL |
| Device Address | 02 |
| Liquid level output value | 021.800 (unit: cm) |
| temperature | 000.000 |
| Protocol version number | 4 Xinde Standard Edition |
| Software version number | 12 |
| Hardware version number | z |
| Number of echoes | 02 |
| Software status code | 0 |
| Hardware status code | 0 |
| Installation status code | 2 |
| Excitation wave multiples | 06 |
| Check code | BF |

Sampledata!AIOIL,02,021.800,000.000,412z,0200,2,06,BF

- *XD format

The data format for this information is: *XD,0000,01,1273,1275,1722,0268,1377#. This means that the device ID is 01, has been powered on for 0 hours, the current liquid level is 127.3 mm, and the temperature is +26.8°C. (A positive temperature is represented as 0268, and a negative temperature is represented as 1268. The highest digit "0" represents positive, and the highest digit "1" represents negative.)

| Field | content | describe |
| --- | --- | --- |
| Special beginning of the protocol | *XD |  |
| Opening duration | 0000 | Opened for 0 hours |

| Device ID | 01 | The device ID number is 01 |
| --- | --- | --- |
| Liquid level value | 1273 | The current liquid level is 127.3 mm |
|  | 1275 |  |
| temperature | 0268 | The temperature is 26.8 degrees, the highest digit "0" represents "positive" and the highest digit "1" represents "negative" |
|  | 1377 |  |

- Server reply information transmission general package

No response from the server

# When	the	type	is	0A,	this	bit	transmits	ICCID	information, hexadecimal number

| IMEI | 8 | For example: If the IMEI number is 123456789123456, the terminal ID is: 0x01 0x23 0x45 0x67 0x89 0x12 0x34 0x56 |
| --- | --- | --- |
| IMSI | 8 | For example: If the IMSI number is 123456789123456, the terminal ID is: 0x01 0x23 0x45 0x67 0x89 0x12 0x34 0x56 |
| ICCID | 10 | Example: If the ICCID number is 12345123456789123456, the terminal ID is: 0X12 0x34 0x51 0x23 0x45 0x67 0x89 0x12 0x34 0x56 |

# When the 94 type is 1E ( temperature sensor ) (supported by BR1201)

2 bytes:

A total of 1 temperature occupies 2 bytes, each temperature occupies 2 bytes unit 0.1 degrees The highest bit represents the sign, for example, 0X8008 represents minus 0.8 degrees Celsius 0X0008 means 0.8 degrees

For example:

0110: First channel: 27.2 degrees Celsius

FFFF: Second channel: invalid data, the temperature sensor is not connected or the sensor is damaged 8016: Fifth channel: (the highest bit is 1, indicating negative temperature) -2.2 degrees Celsius

Original data: 79 79 00 08 94 1E 00 FA 00 58 FE 5A 0D 0A

Packet Type: General Information Transfer Packet (94) Packet length: 8 (0008)

Information type: Temperature sensor (1E) Channel 1 Temperature: 25.0°C (00FA) Message sequence number: 88 (0058) Verification information: FE5A (FE5A)

# When the 94 type is 1F ( oil level sensor ) (supported by BR1201)

4 bytes:

Second channel oil level, 4 bytes, can represent two analog quantities

For example: 64050 ohms (the original AD value of the oil level sensor) is 0XFA32 (oil level 1), 0XFA32 (oil level 2)

(Note: This AD value is the AD value of 000000~~00FFFF corresponding to the oil level change from 0% to 100%)

(FA32=64050, 64050/65535= 97.7% oil volume)

For example:

Original data: 79 79 00 0A 94 1F 05 55 00 00 00 50 B8 05 0D 0A

Packet type: General information transmission packet (94) Packet length: 10 (000A)

Information type: Oil level sensor (1F)

Single channel oil level and corresponding percentage: 1365, 2.08% (0555) Dual channel oil level and corresponding percentage: 0, 0.0% (0000) Message sequence number: 80 (0050)

Verification information: B805 (B805)

Original data: 79 79 00 0A 94 1F 00 00 00 1B 00 63 50 9F 0D 0A

Packet type: General information transmission packet (94) Packet length: 10 (000A)

Information type: Oil level sensor (1F)

Single channel oil level and corresponding percentage: 0, 0.0% (0000) Dual channel oil level and corresponding percentage: 27, 0.04% (001B) Message sequence number: 99 (0063)

Verification information: 509F (509F)

# External module transparent transmission protocol [0X9B]

    - Used to transparently transmit plug-in module data The server sends a data packet to the module

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x9B |
| Informati on content | Module type code | 1 | Used to identify the module type. Different module types have different meanings of transparent data (the corresponding list is as follows) |
|  | Transparent data | N | Transparent data |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: To be supplemented

| Module type code |  |  | illustrate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0x01 |  |  |  |  |  |  |  |
| 0x02 |  |  |  | RFID |  |  |  |  |
| 0x03 |  |  | Serial port transparent transmission |  |  |  |  |  |
| 0x04 |  |  | Feibo |  |  |  |  |  |
| 0x0 5 |  |  |  | The first serial port |  |  |  |  |
|  |  |  |  | transparent |  |  |  |  |
|  |  |  |  | transmission |  |  |  |  |
| 0x0 6 |  |  |  | Second serial port |  |  |  |  |
|  |  |  |  | transparent |  |  |  |  |
|  |  |  |  | transmission |  |  |  |  |
| 0x07 |  |  | Inertial navigation UBI protocol |  |  |  |  |  |
| 0x08 |  |  | N B alarm upload module |  |  |  |  |  |

SGBRAS RFID data

(tentative)

ibutton data reporting

Terminal log reporting

reporting (tentative)

Customer-customized

0x09

0X13

0X14

The module replies data to the server

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x9B |
| Informati on content | Module type code | 1 | Reply the same as sent |
|  | Transparent data | N | Transparent data |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Example data:

Original data::79 79 00 14 9B 02 02 31 42 30 30 31 33 46 37 37 37 38 38 03 00 0B F8 E6 0D 0A

Data packet type: External module transparent transmission protocol (9B) Data packet length: 20(0014)

External module code: RFID information reporting (02)

Transparent data: 0231423030313346373737383803(0231423030313346373737383803)

Start byte: 02(02)

RFID ID: 1B0013F777(31423030313346373737) RFID checksum: 3838(3838)

End byte: 03(03)

Information serial number: 000B (000B) Checksum: F8E6(F8E6)

Special protocol: This protocol is 9B, but it is uploaded from the terminal to the server

#### 0x02 RFID information reporting (RFID transparent transmission protocol)

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
|  | Module type code | 1 | 0x02 RFID information reporting |
| Information | Transparent | n | RFID data 79 79 00 14 9B 02 02 31 42 30 30 31 33 46 37 37 37 38 38 03 00 0B F8 E6 |
| content | data |  | 0D 0A RFID card NO. 31 42 30 30 31 33 46 37 37 37(hexadecimal convert to ASCII) |

# RFID sample data:

Original data: 79 79 00 14 9B 02 02 31 42 30 30 31 33 46 37 37 37 38 38 03 00 0B F8 E6 0D 0A

ↆ Package type: External module transparent transmission protocol (9B) Packet length: 20(0014)

External module coding: RFID information reporting (02)

Transparent data: 0231423030313346373737383803(0231423030313346373737383803)

Start bit: 02(02)

RFID number: 1B0013F777(31423030313346373737) RFID verification code: 3838(3838)

End bit: 03(03)

Message serial number: 000B(000B) Verification information: F8E6(F8E6)

# IBUTTON example:

Original data: 79 79 00 0E 9B 13 01 18 91 30 01 00 00 5E 00 06 B9 F3 0D 0A

Packet type: plug-in module transparent transmission protocol (9B) Packet length: 14 (000E)

Transmitted data type: ibutton (13) Transparent data: 011891300100005E Message sequence number: 6 (0006) Verification information: B9F3 (B9F3)

  - External module transparent transmission protocol (module sending) [0X9C]

    - Used to transparently transmit plug-in module data

The module sends data packets to the server

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x9C |
| Informati on content | Module type code | 1 | Used to identify module types. Different module types have different meanings of transparent data (the corresponding list is the same as 9B) |
|  | Transparent data | N | Transparent data |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

The server responds with data to the module

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x9C |
| Informati on content | Module type code | 1 | Reply the same as sent |
|  | Transparent data | N | Transparent data |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC |

|  |  |  | error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
| --- | --- | --- | --- | --- |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |  |

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x9C |
| Informatio n content | Module type code | 1 | Used to identify module types. Different module types have different meanings of transparent data (the corresponding list is the same as 9B) |
|  | Transpar ent data | N | Transparent data |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Information content:

| Information content | Modul e type code | 1 | - |
| --- | --- | --- | --- |
|  | Transp arent data | - | - |

The type is 1D. This bit transmits part of the original alarm data information in hexadecimal.

|  |  | length |  | Detailed explanation |
| --- | --- | --- | --- | --- |
| Alarm Type |  | 1 |  | 0x01 Collision alarm (xyz axis is greater than the threshold) 0x02 Free fall alarm (based on the algorithm of gravity acceleration ) |
| Time when the alarm |  | 6 |  | Year (1 byte) Month (1 byte) Day (1 |
| was generated |  |  |  | byte) Hour (1 byte) Minute (1 byte) |
|  |  |  |  | Second (1 byte) (convert to decimal) |
| (i.e. the time of the |  |  |  |  |
| original data 0 ) |  |  |  |  |
| Original data0 (Data collected at the time of alarm) |  | S t a t u s | 1 | Convert to a 16-bit binary number and calculate based on the bits (see the table below for details) |
|  |  | B i t s |  |  |
|  |  | D i m e n s i o n s | 4 | Convert to decimal and divide by 1800000 to get the result |

|  |  | l 4 o n g i t u d e | Convert to decimal and divide by 1800000 to get the result |  |
| --- | --- | --- | --- | --- |
|  |  | s 1 p e e d | Convert to decimal to get the result ( km/h ) |  |
|  |  | X 2 - a x i s | Convert to decimal to get the result ( High byte first, low byte last; both positive and negative ) |  |
|  |  | Y 2 - a x i s |  |  |
|  |  | Z 2 - a x i s |  |  |

| The starting time of the original data (ie the time of original data 1 ) |  | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |  |
| --- | --- | --- | --- | --- |
| Original data 1 |  | S 1 t a t u s B i t s | Convert to a 16-bit binary number and calculate based on the bits (see the table below for details) |  |
|  |  | D 4 i m e n s i o n s | Convert to decimal and divide by 1800000 to get the result |  |
|  |  | l 4 o n g i t u d e | Convert to decimal and divide by 1800000 to get the result |  |

|  |  | s 1 p e e d | Convert to decimal to get the result |  |
| --- | --- | --- | --- | --- |
|  |  | X 2 - a x i s | Convert to decimal to get the result ( High byte first, low byte last; both positive and negative ) |  |
|  |  | Y 2 - a x i s |  |  |
|  |  | Z 2 - a x i s |  |  |
| ...... |  | . ...... . . . . . | ...... |  |
| Original |  | S 1 t | Convert to a 16-bit binary number and calculate based on the bits (see the |  |

| data 60 |  | a t u s B i t s | table below for details) |  |
| --- | --- | --- | --- | --- |
|  |  | D 4 i m e n s i o n s | Convert to decimal and divide by 1800000 to get the result |  |
|  |  | l 4 o n g i t u d e | Convert to decimal and divide by 1800000 to get the result |  |
|  |  | s 1 p e e d | Convert to decimal to get the result |  |

|  |  | X 2 - a x i s | Convert to decimal to get the result (High byte first, low byte last; both positive and negative) |  |
| --- | --- | --- | --- | --- |
|  |  | Y 2 - a x i s |  |  |
|  |  | Z 2 - a x i s |  |  |

Note: X, Y, Z axis: high byte first, low byte last; high byte 1 indicates negative number, 0 indicates positive number. Example: Convert to 16-bit binary;

The highest bit is 1, indicating a positive number, which is converted to decimal as the value of the x-axis.

The highest bit is 0, indicating a negative number. Take the two's complement, add 1, and add a "-" sign to get the value of the x-axis.

Data example: 0004, X-axis data is 4, FF04, X-axis data is -252

# ------The following is the reserved agreement: (currently not in use)

Differentiated login package [0X01]

#### Login Packet

|  | 2. |  | length | Detailed explanation |
| --- | --- | --- | --- | --- |
|  | Start bit |  | 2 | 0x78 0x78 |
|  | Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
|  | Protocol Number |  | 1 | 0x01 |
|  | Information content | Terminal ID | 8 | For example: If the IMEI number is 123456789123456, the terminal ID is: 0x01 0x23 0x45 0x67 0x89 0x12 0x34 0x56 |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Example data: 78 78 0D 01 03 53 41 90 33 41 28 36 00 0D 33 51 0D 0A

#### Login packet reply (server reply)

| 3. | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x01 |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 05 01 00 05 9F F8 0D 0A

# Differentiated positioning package [0X12]

#### The terminal sends a positioning packet

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x12 (The time zone of the machine is set) |

|  | Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) (time zone) |
| --- | --- | --- | --- | --- |
|  |  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  |  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  |  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  |  | speed | 1 | Convert to decimal to get the result |
|  |  | Heading, status | 2 | Convert to a 16-bit binary number and perform calculations based on the bits (see the table below for details) |
|  |  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  |  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  |  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  |  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 1F 12 0F 0C 1D 0B 0F 34 C6 02 7A C7 4C 0C 46 58 10 00 14 D4 01 CC 00 28 7D

00 1F 71 00 26 23 09 0D 0A

ii.	Detailed explanation of status heading

| BYTE_1 | Bit7 | 0 |
| --- | --- | --- |
|  | Bit6 | 0 |
|  | Bit5 | GPS real-time/differ ential positioning |
|  | Bit4 | GPS positioning has been |
|  | Bit3 | East longitude, West longitude |
|  | Bit2 | South latitude, north latitude |
|  | Bit1 | course |
|  | Bit0 |  |
| BYTE_2 | Bit7 |  |

It occupies 2 bytes and indicates the direction of GPS operation. The range is 0 to 360 degrees. The unit is degree, with due north as 0 degrees and clockwise.

Bit0

Bit1

Bit2

Bit3

Bit4

Bit5

Bit6

#### Server Location Packet Reply

The positioning packet server does not need to reply

# Differentiated LBS package [0X18]

#### LBS multi-base station extended information package

of LBS multi-base station extended information package :

  - Data packets used to transmit the location of the terminal when it is not positioned

#### The terminal sends LBS multi-base station package

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x18 |
| Information content | Date and time (UTC) | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  | CI | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | RSSI | 1 | Cell signal strength, the value range is 0x00 ~ 0xFF, 0x00 is the weakest signal, 0xFF is the strongest signal. |
|  | NLAC1 | 2 | Same as above LAC |
|  | NCI1 | 3 | Same as above CI |
|  | NRSSI1 | 1 | Same as RSSI |
|  | NLAC2 | 2 | Same as above LAC |
|  | NCI2 | 3 | Same as above CI |
|  | NRSSI2 | 1 | Same as RSSI |
|  | NLAC3 | 2 | Same as above LAC |
|  | NCI3 | 3 | Same as above CI |
|  | NRSSI3 | 1 | Same as RSSI |
|  | NLAC4 | 2 | Same as above LAC |
|  | NCI4 | 3 | Same as above CI |
|  | NRSSI4 | 1 | Same as RSSI |
|  | NLAC5 | 2 | Same as above LAC |
|  | NCI5 | 3 | Same as above CI |
|  | NRSSI5 | 1 | Same as RSSI |
|  | NLAC6 | 2 | Same as above LAC |
|  | NCI6 | 3 | Same as above CI |
|  | NRSSI6 | 1 | Same as RSSI |
|  | Time advance | 1 | It refers to the difference between the actual time it takes for a mobile station signal to arrive at the base station and the time it takes for the mobile station signal to arrive at the base station assuming the distance between the mobile station and the base |

|  |  |  |  | station is 0. |  |
| --- | --- | --- | --- | --- | --- |
|  |  | language | 2 | 0x00 0x01 Chinese 0x00 0x02 English |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

Sample data: 78 78 3B 1 8 10 01 0D 02 02 02 01 CC 00 28 7D 00 1F 71 3E 28 7D 00 1F 72 31 28

7D 00 1E 23 2D 28 7D 00 1F 40 18 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 FF 00

02 00 05 B1 4B 0D 0A

#### Server LBS multi-base station package reply

LBS multi-base station package server does not need to reply

# Differentiated alarm package [0X16]

#### Alarm package

of the alarm package:

  - Used to transmit terminal-defined alarm content

  - After receiving the alarm content, the server responds and resolves the longitude and latitude into an address and sends it back to the terminal

  - The terminal sends the returned address to the SOS number set on the terminal

#### The terminal sends an alarm packet

Alarm package

| iii. |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x16 |
| Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) (time zone) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |

|  |  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
| --- | --- | --- | --- | --- |
|  |  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  |  | speed | 1 | Convert to decimal to get the result |
|  |  |  |  | Convert to 16-bit binary number and perform calculation based |
|  |  | Heading, status | 2 | on the bit (same as GPS package, see GPS package explanation |
|  |  |  |  | for details) |
|  |  | LBS length | 1 | Total length of LBS information (length + MCC + MNC + CellID) |
|  |  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  |  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  |  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  |  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  |  | Terminal Information | 1 | See the table below for details |
|  |  |  |  | 0x00: No power (powered off) |
|  |  |  |  | 0x01: Very low power (not enough to make calls or send text |
|  |  |  |  | messages, etc.) |
|  |  | Voltage level | 1 | 0x02: Very low power (low power alarm) 0x03: |
|  |  |  |  | Low power (can be used normally) 0x04: Medium power |
|  |  |  |  | 0x05: High power |
|  |  |  |  | 0x06: Very high power |
|  |  |  |  | 0x00: No signal; |
|  |  |  |  | 0x01: Very weak signal |
|  |  | GSM signal level | 1 | 0x02: Weak signal |
|  |  |  |  | 0x03: Good signal |
|  |  |  |  | 0x04: Strong signal |
|  |  | Alarm, language | 2 | See the table below for details |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  |  |  |  | The CRC-ITU value from "Packet Length" to "Message Sequence |
|  | Error checking |  | 2 | Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be |
|  |  |  |  | discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 25 16 0F 0C 1D 0A 2B 21 C8 02 7A C8 04 0C 46 58 10 00 14 6F 09 01 CC 00 28

7D 00 1F 71 48 04 04 03 01 00 1C 84 CF 0D 0A

  - Detailed explanation of terminal information

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric |

|  |  |  | connected |  |
| --- | --- | --- | --- | --- |
|  |  | Bit6 | 1: GPS positioning |  |
|  |  |  | 0: GPS not positioned |  |
|  |  | Bit3～Bit5 | 100: SOS |  |
|  |  |  | 011: Low battery alarm |  |
|  |  |  | 010: Power failure alarm |  |
|  |  |  | 001: Vibration alarm |  |
|  |  |  | 000: Normal |  |
|  |  | Bit2 | 1: Connected to power and charging |  |
|  |  |  | 0: Not connected to a power source for charging |  |
|  |  | Bit1 | 1: ACC high |  |
|  |  |  | 0: ACC low |  |
|  |  | Bit0 | 1: Fortification |  |
|  |  |  | 0: Disarm |  |

  - Detailed explanation of alarm language

| Byte 1 | 0x00: Normal |
| --- | --- |
|  | 0x01: SOS |
|  | 0x02: Power failure alarm |
|  | 0x03: Vibration alarm |
|  | 0x04: Fence entry alarm |
|  | 0x05: Fence-out alarm |
|  | 0x06 Overspeed Alarm |
|  | 0x09 Displacement Alarm |
|  | 0x0A Entering GPS blind area alarm |
|  | 0x0B GPS blind zone alarm |
|  | 0x0C Power-on alarm |
|  | 0x0D GPS first positioning alarm |
|  | 0x0E External low power alarm |
|  | 0x0F External low power protection alarm |
|  | 0x10 Card replacement alarm |
|  | 0x11 Shutdown Alarm |
|  | 0x12 Flight mode alarm after external low battery protection |
|  | 0x13 Disassembly Alarm |
|  | 0x14 Door /digital input alarm |
| Byte 2 | 0x01 Chinese |

|  |  | 0x02 English 0x00 No platform response required |  |
| --- | --- | --- | --- |

#### Server alarm packet reply

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x16 |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

Example data: 78 78 05 16 00 1C 1B 28 0D 0A

# Differentiated command reply packet [0X15]

#### Terminal online command reply

Terminal reply (general command)

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x7 8 0x7 8 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x 15 |
| Information content | Command length | 1 | Server flag + command content length |
|  | Server flags | 4 | The server will use it to identify the data. The terminal will return the received data in binary format in the return packet. |
|  | content | M | The data to be sent (according to the content encoding format) |
|  | language | 2 | Chinese: 0x00 0x01 English: 0x00 0x02 |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it |

|  |  |  | will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| --- | --- | --- | --- |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

Example: 78 78 28 15 20 000 000 000 53 4F 53 31 3A 31 33 34 32 31 36 33 32 36 39 39 20 53 4F 53 32 3A 20 53 4F 53

33 3A 00 01 00 2A C3 9C 0D 0A

## WIFI interest package [0X2C]

of WIFI information package :

  - Used to transmit WIFI data packets received by the terminal

- The terminal sends a WiFi packet

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x2C |
| Informati on content | Date and time (UTC) | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | MCC | 2 | Mobile Country Code |
|  | MNC | 1 | Mobile Network Code (MNC) |
|  | LAC | 2 | Mobile Network Code (MNC) |
|  | CI | 3 | Cell Tower ID (Cell ID) |
|  | RSSI | 1 | Cell signal strength, the value range is 0x00 ~ 0xFF, 0x00 is the weakest signal, 0xFF is the strongest signal. |
|  | NLAC1 | 2 | Same as above LAC |
|  | NCI1 | 3 | Same as above CI |
|  | NRSSI1 | 1 | Same as RSSI |
|  | NLAC2 | 2 | Same as above LAC |
|  | NCI2 | 3 | Same as above CI |
|  | NRSSI2 | 1 | Same as RSSI |
|  | NLAC3 | 2 | Same as above LAC |
|  | NCI3 | 3 | Same as above CI |
|  | NRSSI3 | 1 | Same as RSSI |
|  | NLAC4 | 2 | Same as above LAC |
|  | NCI4 | 3 | Same as above CI |
|  | NRSSI4 | 1 | Same as RSSI |
|  | NLAC5 | 2 | Same as above LAC |
|  | NCI5 | 3 | Same as above CI |
|  | NRSSI5 | 1 | Same as RSSI |
|  | NLAC6 | 2 | Same as above LAC |
|  | NCI6 | 3 | Same as above CI |
|  | NRSSI6 | 1 | Same as RSSI |
|  | Time advance | 1 | It refers to the difference between the actual time it takes for a mobile station signal to reach the base station and the time it takes for the mobile |

|  |  |  |  | station signal to reach the base station assuming the distance between the mobile station and the base station is 0. |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Wi -Fi quantity | 1 | Used to determine the number of WIFIs transmitted in the packet, 0 means no WIFI is detected |  |
|  |  | WIFI MAC1 | 6 | Received signal 1 WIFI MAC (transmitted based on the actual number of WIFIs found, if one is found, one is transmitted; if multiple are found, multiple are transmitted; if no WIFI is found, the transmission is 0) |  |
|  |  | Wi-Fi strength 1 | 1 | Signal 1WIFI signal strength |  |
|  |  | WIFI MAC2 | 6 | Same as above |  |
|  |  | Wi-Fi Strength 2 | 1 | Same as above |  |
|  |  | . . . |  | . . . . |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

Example data: 78 78 48 2C 10 06 0E 02 2D 35 01 CC 00 28 7D 00 1F 71 2D 28 7D 00 1E 17 25 28

7D 00 1E 23 1E 28 7D 00 1F 72 1C 28 7D 00 1F 40 12 00 00 00 00 00 00 00 00 00 00 00 00 FF 02

80 89 17 44 98 B4 5C CC 7B 35 36 61 A6 5B 00 1F A0 04 0D 0A

- Server WIFI packet reply

WIFI packet server does not need to reply

## GPS address request packet [0X1A]

Description of the address request packet:

  - The client sends an address request instruction to the terminal, and the terminal then sends an address request packet to the server to request address resolution.

  - The terminal forwards the address returned by the server to the client

- Terminal address request packet

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x1A |
| Information content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) (time zone) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | speed | 1 | Convert to decimal to get the result |
|  | Heading, status | 2 | Convert to 16-bit binary number and perform calculation based on the bit (same as GPS package, see GPS package explanation for details) |
|  | telephone number | twenty one | telephone number |
|  | Alarm, language | 2 | The last bit is 0x01 Chinese and 0x02 English |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 2E 1A 0F 0C 1D 0A 26 0A C7 02 7A C7 E0 0C 46 57 F0 02 14 72 38 36 31 33 34

32 31 36 33 32 36 39 39 00 00 00 00 00 00 00 00 00 01 00 16 17 4E 0D 0A

- Chinese reply to server address request packet

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |

|  | Protocol Number |  | 1 | 0x17 |  |
| --- | --- | --- | --- | --- | --- |
|  | Information content | Instruction length | 1 | The length of data from the server flag to the message sequence number |  |
|  |  | Server flags | 4 | The server is used to mark the alarm |  |
|  |  | ADDRESS | 8 | Address Request Encoding Flag (ASCII) |  |
|  |  | && | 2 | Separator (ASCII) |  |
|  |  | Address content | M | The address resolved by the server (UNICODE) |  |
|  |  | && | 2 | Separator (ASCII) |  |
|  |  | telephone number | twenty one | The server returns the phone number of the terminal request packet (ASCII) |  |
|  |  | ## | 2 | Separator (ASCII) |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

Example data: 78 78 6E 17 68 00 00 00 01 41 44 44 52 45 53 53 26 26 4F 4D 7F 6E 00 3A 5E 7F

4E 1C 77 01 00 2E 60 E0 5D DE 5E 02 00 2E 60 E0 57 CE 53 3A 00 2E 4E 91 5C 71 89 7F 8D EF 00

2E 79 BB 60 E0 5D DE 5E 02 5B 66 59 27 65 59 80 B2 7E A6 00 32 00 35 7C 73 00 2E 26 26 38 36

31 33 34 32 31 36 33 32 36 39 39 00 00 00 00 00 00 00 00 23 23 00 16 C1 EC 0D 0A

- Server address request packet English reply

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x97 |
| Information content | Instruction length | 1 | The length of data from the server flag to the message sequence number |
|  | Server flags | 4 | The server is used to mark the alarm |
|  | ADDRESS | 8 | Address Request Encoding Flag (ASCII) |
|  | && | 2 | Separator (ASCII) |
|  | Address content | M | The address resolved by the server (UNICODE) |
|  | && | 2 | Separator (ASCII) |
|  | telephone number | twenty one | The server returns the phone number of the terminal request packet (ASCII) |
|  | ## | 2 | Separator (ASCII) |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically |

|  |  |  | increased by 1 |  |
| --- | --- | --- | --- | --- |
|  | Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |  |

00 61 00 6C 00 61 00 72 00 6D 00 2E 00 68 00 74 00 74 00 70 00 3A 00 2F 00 2F 00 6D 00 61 00

70 00 73 00 2E 00 67 00 6F 00 6F 00 67 00 6C 00 65 00 2E 00 63 00 6F 00 6D 00 2F 00 6D 00 61

00 70 00 73 00 3F 00 71 00 3D 00 4E 00 32 00 32 00 2E 00 35 00 37 00 33 00 35 00 36 00 2C 00

45 00 31 00 31 00 33 00 2E 00 39 00 32 00 31 00 37 00 31 26 26 38 36 31 33 34 32 31 36 33 32

36 39 39 00 00 00 00 00 00 00 00 23 23 00 16 8E A5 0D 0A

## LBS address request packet [0X17]

Description of the address request packet:

  - The client sends an address request instruction to the terminal, and the terminal then sends an address request packet to the server to request address resolution.

  - The terminal forwards the address returned by the server to the client

- Terminal address request packet

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x 17 |
| Information content | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 | Mobile Network Code (MNC) (converted to decimal) |
|  | LAC | 2 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 3 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | telephone number | twenty one | telephone number |
|  | Alarm, language | 2 | The last bit is 0x01 Chinese and 0x02 English |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 78 78 24 17 01 CC 00 28 7D 00 1F 71 31 32 35 32 30 31 33 35 33 32 31 37 37 30 37

39 00 00 00 00 00 00 01 00 2A 7D D6 0D 0A

- Chinese reply to server address request packet

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x17 |
| Information content | Instruction length | 1 | The length of data from the server flag to the message sequence number |
|  | Server flags | 4 | The server is used to mark the alarm |
|  | ADDRESS | 8 | Address Request Encoding Flag (ASCII) |
|  | && | 2 | Separator (ASCII) |
|  | Address | M | The address resolved by the server (UNICODE) |

|  |  | content |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | && | 2 | Separator (ASCII) |  |
|  |  | telephone number | twenty one | The server returns the phone number of the terminal request packet (ASCII) |  |
|  |  | ## | 2 | Separator (ASCII) |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

Example data: 78 78 6E 17 68 00 00 00 01 41 44 44 52 45 53 53 26 26 4F 4D 7F 6E 00 3A 5E 7F

4E 1C 77 01 00 2E 60 E0 5D DE 5E 02 00 2E 60 E0 57 CE 53 3A 00 2E 4E 91 5C 71 89 7F 8D EF 00

2E 79 BB 60 E0 5D DE 5E 02 5B 66 59 27 65 59 80 B2 7E A6 00 32 00 35 7C 73 00 2E 26 26 38 36

31 33 34 32 31 36 33 32 36 39 39 00 00 00 00 00 00 00 00 23 23 00 16 C1 EC 0D 0A

- Server address request packet English reply

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x97 |
| Information content | Instruction length | 1 | The length of data from the server flag to the message sequence number |
|  | Server flags | 4 | The server is used to mark the alarm |
|  | ADDRESS | 8 | Address Request Encoding Flag (ASCII) |
|  | && | 2 | Separator (ASCII) |
|  | Address content | M | The address resolved by the server (UNICODE) |
|  | && | 2 | Separator (ASCII) |
|  | telephone number | twenty one | The server returns the phone number of the terminal request packet (ASCII) |
|  | ## | 2 | Separator (ASCII) |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

00 61 00 6C 00 61 00 72 00 6D 00 2E 00 68 00 74 00 74 00 70 00 3A 00 2F 00 2F 00 6D 00 61 00

70 00 73 00 2E 00 67 00 6F 00 6F 00 67 00 6C 00 65 00 2E 00 63 00 6F 00 6D 00 2F 00 6D 00 61

00 70 00 73 00 3F 00 71 00 3D 00 4E 00 32 00 32 00 2E 00 35 00 37 00 33 00 35 00 36 00 2C 00

45 00 31 00 31 00 33 00 2E 00 39 00 32 00 31 00 37 00 31 26 26 38 36 31 33 34 32 31 36 33 32

36 39 39 00 00 00 00 00 00 00 00 23 23 00 16 8E A5 0D 0A

Information transmission package [0X98] ( 98 package, only used by some projects )

- Terminal sends to server

|  |  | lengt h | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x98 |
| Informati on content | Module No. 1 | 1 | Module number |
|  | Module length 1 | 2 | Module content length |
|  | Module Content 1 | M | Determine module content information based on module number |
|  | Module No. 2 | 1 | Module number |
|  | Module length 2 | 2 | Module content length |
|  | Module Content 2 | M | Determine module content information based on module number |
|  | . . . |  | . . . . . . . |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 79 79 00 28 98 00 00 08 08 68 12 01 48 37 35 71 01 00 08 04 60 04 03 40 00 99 32

02 00 0A 89 86 02 B3 13 15 90 10 99 32 00 04 F5 81 0D 0A

Module number details

| 0x00 | IMEI (hexadecimal) | 0x10 | SN of the whole machine |
| --- | --- | --- | --- |
| 0x01 | IMSI (hexadecimal) | 0x11 | Battery SN |
| 0x02 | ICCID (hexadecimal) | 0x12 | Motor SN |
| 0x03 | Chip ID (hexadecimal) | 0x13 | Real-time data ( 9 bytes) |
| 0x04 | Bluetooth MAC (hexadecimal) | 0x14 | Real-time status ( 2 bytes) |
| 0x05 | Password | 0x15 | Fault information ( 2 bytes) |
| 0x06 | Secretkey | 0x16 | Vehicle setting information ( 6 bytes) |
| 0x07 | s oftVersion | 0x17 | Vehicle fixed information (16 bytes ) |

|  | 0x08 | User ID | 0x18 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0x09 |  | 0x19 |  |  |
|  | 0x0A |  | 0x1A |  |  |
|  | 0x0B |  | 0x1B |  |  |
|  | 0x0C |  | 0x1C |  |  |
|  | 0x0D |  | 0x1D |  |  |
|  | 0x0E |  | 0x1E |  |  |
|  | 0x0F |  | 0x1F |  |  |

Detailed explanation of module number content

- The server replies to the terminal

|  | length | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x79 0x79 |
| Packet length | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x98 |
| Reserved extension bit length | 1 | This bit is added for future function expansion. If this bit is 0, the reserved expansion bit is empty. |
| Reserved extension bit | N | Function extension bit. If the reserved extension bit length is 0, this bit is empty. |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

Sample data: 79 79 00 06 98 00 00 00 C7 00 0D 0A

## 4G GPS positioning package (4G base station, protocol number: 0xA0)

Description of GPS positioning package:

  - Data packets used to transmit the terminal's location

  - After positioning and connection are successful, upload positioning data points according to the set rules

  - If the connection is successful and there is a cached positioning point, the previously located data point will be retransmitted.

|  |  | lengt h | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0xA0 (UTC) |
| Informati on content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | speed | 1 | Convert to decimal to get the result |
|  | Heading, status | 2 | Convert to a 16-bit binary number and perform calculations based on the digits (see Appendix 3 for details) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) See note below for length |
|  | LAC | 4 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 8 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | ACC | 1 | ACC status ACC low is 00, ACC high is 01 (06 does not have this function) |
|  | Data reporting mode | 1 | GPS data point reporting type (06 does not have this function) 0x00 Regular reporting 0x01 Fixed-distance reporting 0x02 Inflection point upload 0x03 ACC status change upload 0x04 After changing from motion to stillness, retransmit the last positioning point 0x05 After the network is disconnected and reconnected, the last valid upload point is reported |

The terminal sends a positioning packet Positioning package

|  |  |  |  | 0X06 reporting mode: ephemeris update forced upload of GPS points 0X07 reporting mode: press the button to upload the positioning point 0X08 reporting mode: reporting location information when powered on 0X09 reporting mode: not used 0X0A reporting mode: the device reports the last latitude and longitude after being stationary, but the time is updated 0X0B WIFI parsing latitude and longitude upload package 0X0C reporting mode: LJDW (immediate positioning) command reporting 0X0D reporting mode: the device reports the last latitude and longitude after being stationary 0X0E reporting mode: GPSDUP upload (timing upload in static state) |
| --- | --- | --- | --- | --- |
|  |  | GPS real-time transmission | 1 | 0x00 real-time upload 0x01 supplementary upload (06 does not have this function) |
|  |  | Mileage statistics | 4 | Convert to decimal to get the result (some products have this function, some do not have this function and there is no such position) |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Note:  To  be  compatible  with  some  countries  where  MNC  occupies  two  bytes  ,  the  highest  bit  of MCC is used to distinguish the MNC length .

When1,theMNClengthis2.ThedefaultvalueofBit15oftheolddeviceshippedis0;thedefault                     value of Bit 15 of the new device is 1.

MCC position detailed explanation

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE S | Bit15 | 1: MNC length is 2 |
|  |  | 0: MNC length is 1 |
|  | Bit0~bit14 | MCC Information |

iv.	Detailed explanation of status heading

| BYTE_1 | Bit7 | 0 |
| --- | --- | --- |
|  | Bit6 | 0 |
|  | Bit5 | GPS |

It occupies 2 bytes and indicates the direction of GPS operation. The range is 0 to 360 degrees. The unit is degree. The north is 0 degrees and the direction is clockwise.

|  |  |  | real-time/differ ential positioning |  |
| --- | --- | --- | --- | --- |
|  |  | Bit4 | GPS positioning has been |  |
|  |  | Bit3 | East longitude, West longitude |  |
|  |  | Bit2 | South latitude, north latitude |  |
|  |  | Bit1 | course |  |
|  |  | Bit0 |  |  |
|  | BYTE_2 | Bit7 |  |  |
|  |  | Bit6 |  |  |
|  |  | Bit5 |  |  |
|  |  | Bit4 |  |  |
|  |  | Bit3 |  |  |
|  |  | Bit2 |  |  |
|  |  | Bit1 |  |  |
|  |  | Bit0 |  |  |

- Server Location Packet Reply

## 4G	LBS multi-base station extended information packet (4G base station, protocol number: 0x A 1)

of LBS multi-base station extended information package :

  - Data packets used to transmit the location of the terminal when it is not positioned

- The terminal sends LBS multi-base station package

|  |  | leng th | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0xA1 |
| Informati on content | Date and time (UTC) | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) See note below for length |
|  | LAC | 4 | Location Area Code (LAC) (converted to decimal) |
|  | CI | 8 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | RSSI | 1 | Cell signal strength, the value range is 0x00 ~ 0xFF, 0x00 is the weakest signal, 0xFF is the strongest signal. |
|  | NLAC1 | 4 | Same as above LAC |
|  | NCI1 | 8 | Same as above CI |
|  | NRSSI1 | 1 | Same as above RSSI |
|  | NLAC2 | 4 | Same as above LAC |
|  | NCI2 | 8 | Same as above CI |
|  | NRSSI2 | 1 | Same as above RSSI |
|  | NLAC3 | 4 | Same as above LAC |
|  | NCI3 | 8 | Same as above CI |
|  | NRSSI3 | 1 | Same as above RSSI |
|  | NLAC4 | 4 | Same as above LAC |
|  | NCI4 | 8 | Same as above CI |
|  | NRSSI4 | 1 | Same as above RSSI |
|  | NLAC5 | 4 | Same as above LAC |
|  | NCI5 | 8 | Same as above CI |
|  | NRSSI5 | 1 | Same as above RSSI |
|  | NLAC6 | 4 | Same as above LAC |
|  | NCI6 | 8 | Same as above CI |

|  |  | NRSSI6 | 1 | Same as above RSSI |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Time advance | 1 | It refers to the difference between the actual time it takes for a mobile station signal to reach the base station and the time it takes for the mobile station signal to reach the base station assuming the distance between the mobile station and the base station is 0. |  |
|  |  | language | 2 | 0x00 0x01 Chinese 0x00 0x02 English |  |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

Note:  To  be  compatible  with  some  countries  where  MNC  occupies  two  bytes  ,  the  highest  bit  of MCC is used to distinguish the MNC length .

When1,theMNClengthis2.ThedefaultvalueofBit15oftheolddeviceshippedis0;thedefault                     value of Bit 15 of the new device is 1.

MCC position detailed explanation

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE S | Bit15 | 1: MNC length is 2 |
|  |  | 0: MNC length is 1 |
|  | Bit0~bit14 | MCC Information |

- Server LBS multi-base station package reply

0x28 LBS multi-base station package server does not need to reply

## 4G single fence alarm package (4G base station, protocol number: 0xA3 / 0xA4 )

of the alarm package:

  - Used to transmit terminal-defined alarm content

  - After receiving the alarm content, the server responds and resolves the longitude and latitude into an address and sends it back to the terminal

  - The terminal sends the returned address to the SOS number set on the terminal

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0xA3 (UTC) |
| Informati on content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | speed | 1 | Convert to decimal to get the result |
|  | Heading, status | 2 | Convert to 16-bit binary number and calculate based on the bit (same as GPS package, see GPS package explanation for details) |
|  | LBS length | 1 | Total length of LBS information (length + MCC + MNC + CellID) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) See note below for length |
|  | LAC | 4 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 8 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Terminal Information | 1 | See the table below for details |
|  | Voltage level | 1 | 0x00: No power (power off) 0x01: The battery is extremely low (not enough to make calls or send text messages, etc.) 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) 0x04: Battery level 0x05: High battery 0x06: Extremely high battery |

The terminal sends an alarm packet Alarm package (single fence)

|  |  | GSM signal level | 1 | 0x00: no signal; 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal 0x04: Strong signal |
| --- | --- | --- | --- | --- |
|  |  | Alarm, language | 2 | See the table below for details |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Alarm package (multi-fence)

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0xA4 (UTC) |
| Informati on content | Date and Time | 6 | Year (1 byte) Month (1 byte) Day (1 byte) Hour (1 byte) Minute (1 byte) Second (1 byte) (convert to decimal) |
|  | Number of GPS information satellites | 1 | The first character is the length of the GPS information, and the second character is the number of satellites involved in positioning (converted to decimal) |
|  | latitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | longitude | 4 | Convert to decimal and divide by 1800000 to get the result |
|  | speed | 1 | Convert to decimal to get the result |
|  | Heading, status | 2 | Convert to 16-bit binary number and calculate based on the bit (same as GPS package, see GPS package explanation for details) |
|  | LBS length | 1 | Total length of LBS information (length + MCC + MNC + CellID) |
|  | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) See note below for length |
|  | LAC | 4 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 8 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Terminal Information | 1 | See the table below for details |

|  |  |  |  | 0x00: No power (power off) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 0x01: The battery is extremely low (not enough to make calls or send |
|  |  |  |  | text messages, etc.) |
|  |  | Voltage level | 1 | 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) |
|  |  |  |  | 0x04: Battery level |
|  |  |  |  | 0x05: High battery |
|  |  |  |  | 0x06: Extremely high battery |
|  |  |  |  | 0x00: no signal; |
|  |  | GSM signal level | 1 | 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal |
|  |  |  |  | 0x04: Strong signal |
|  |  | Alarm, language | 2 | See the table below for details |
|  |  | Fence number | 1 | Fence alarm: This bit is valid, 1 is fence 1, 2 is fence 2 , and FF is invalid. |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  |  |  |  | The CRC-ITU value from "Packet Length" to "Message Sequence Number". |
|  | Error checking |  | 2 | If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix |
|  |  |  |  | 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

Note:  To  be  compatible  with  some  countries  where  MNC  occupies  two  bytes  ,  the  highest  bit  of MCC is used to distinguish the MNC length .

When1,theMNClengthis2.ThedefaultvalueofBit15oftheolddeviceshippedis0;thedefault                     value of Bit 15 of the new device is 1.

MCC position detailed explanation

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE S | Bit15 | 1: MNC length is 2 |
|  |  | 0: MNC length is 1 |
|  | Bit0~bit14 | MCC Information |

  - Detailed explanation of terminal information

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric connected |

|  |  | Bit6 | 1: GPS positioning |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0: GPS not positioned |  |
|  |  | Bit3～Bit5 | 100: SOS |  |
|  |  |  | 011: Low battery alarm |  |
|  |  |  | 010: Power failure alarm |  |
|  |  |  | 001: Vibration alarm |  |
|  |  |  | 000: Normal |  |
|  |  | Bit2 | 1: Connected to power and charging |  |
|  |  |  | 0: Not connected to a power source for charging |  |
|  |  | Bit1 | 1: ACC high |  |
|  |  |  | 0: ACC low |  |
|  |  | Bit0 | 1: Fortification |  |
|  |  |  | 0: Disarm |  |

  - Detailed explanation of alarm language

| Byte 1 | 0x00: Normal |
| --- | --- |
|  | 0x01: SOS |
|  | 0x02: Power failure alarm |
|  | 0x03: Vibration alarm |
|  | 0x04: Fence entry alarm |
|  | 0x05: Fence-out alarm |
|  | 0x06 Overspeed Alarm |
|  | 0x09 Displacement Alarm |
|  | 0x0A Entering GPS blind area alarm |
|  | 0x0B GPS blind zone alarm |
|  | 0x0C Power-on alarm |
|  | 0x0D GPS first positioning alarm |
|  | 0x0E External low power alarm |
|  | 0x0F External low power protection alarm |
|  | 0x10 Card replacement alarm |
|  | 0x11 Shutdown Alarm |
|  | 0x12 Flight mode alarm after external low battery protection |
|  | 0x13 Disassembly Alarm |
|  | 0x14 Door Alarm |
|  | 0x15 Low power shutdown alarm |
|  | 0x16 Voice-activated alarm |

|  |  | 0x17 Fake Base Station Alarm |  |
| --- | --- | --- | --- |
|  |  | 0x18 Cover open alarm |  |
|  |  | 0x19 Internal battery low power alarm |  |
|  |  | 0x20 Entering deep sleep alarm |  |
|  |  | 0x21 Reserved |  |
|  |  | 0x22 Reserved |  |
|  |  | 0x23 Fall Alarm |  |
|  |  | 0xFF ACC off |  |
|  |  | 0xFE ACC open |  |
|  | Byte 2 | 0x01 Chinese 0x02 English 0x00 No platform response required |  |

- Server alarm packet reply

|  | lengt h | Detailed explanation |
| --- | --- | --- |
| Start bit | 2 | 0x78 0x78 |
| Packet length | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number | 1 | 0x26 (UTC) |
| Information sequence number | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits | 2 | Fixed value, unified as 0x0D 0x0A |

- Server alarm package Chinese address reply

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x17 |
| Informati on content | Instruction length | 1 | The length of data from the server flag to the message sequence number |
|  | Server flags | 4 | The server is used to mark the alarm |
|  | ALARMSMS | 8 | Alarm code mark (ASCII) |
|  | && | 2 | Alarm code mark (ASCII) |
|  | Address content | M | The address resolved by the server (UNICODE) |

|  |  | && | 2 | Separator (ASCII) |
| --- | --- | --- | --- | --- |
|  |  | telephone number | twenty one | When uploading the alarm package, transmit all "0" (ASCII) |
|  |  | ## | 2 | Separator (ASCII) |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

- Server alarm package English address reply

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 2 | 0x97 |
| Informati on content | Instruction length | 1 | The length of data from the server flag to the message sequence number |
|  | Server flags | 4 | The server is used to mark the alarm |
|  | ALARMSMS | 8 | Alarm code mark (ASCII) |
|  | && | 2 | Alarm code mark (ASCII) |
|  | Address content | M | The address resolved by the server (UNICODE) |
|  | && | 2 | Separator (ASCII) |
|  | telephone number | twenty one | When uploading the alarm package, transmit all "0" (ASCII) |
|  | ## | 2 | Separator (ASCII) |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

## 4G LBS alarm package (4G base station, protocol number: 0xA5)

of the alarm package:

  - Used to transmit terminal-defined alarm content

  - After receiving the alarm content, the server responds and resolves the LBS base station information into an address and sends it back to the terminal.

  - The terminal sends the returned address to the SOS number set on the terminal

|  |  | length | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x78 0x78 |
| Packet length |  | 1 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0xA5 |
| Informati on content | MCC | 2 | Mobile Country Code (MCC) (converted to decimal) |
|  | MNC | 1 ( or 2 ) | Mobile Network Code (MNC) See note below for length |
|  | LAC | 4 | Location Area Code (LAC) (converted to decimal) |
|  | Cell ID | 8 | Mobile base station Cell Tower ID (Cell ID) (converted to decimal) |
|  | Terminal Information | 1 | See the table below for details |
|  | Voltage level | 1 | 0x00: No power (power off) 0x01: The battery is extremely low (not enough to make calls or send text messages, etc.) 0x02: Lighting is very low (low battery alarm) 0x03: Low battery (can be used normally) 0x04: Battery level 0x05: High battery 0x06: Battery level is extremely high |
|  | GSM signal level | 1 | 0x00: no signal; 0x01: Extremely weak signal 0x02: Weak signal 0x03: Good signal 0x04: Strong signal |
|  | Alarm, language | 2 | See the table below for details |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

The terminal sends an alarm packet Alarm package

Note:  To  be  compatible  with  some  countries  where  MNC  occupies  two  bytes  ,  the  highest  bit  of MCC is used to distinguish the MNC length .

When1,theMNClengthis2.ThedefaultvalueofBit15oftheolddeviceshippedis0;thedefault                     value of Bit 15 of the new device is 1.

MCC position detailed explanation

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE S | Bit15 | 1: MNC length is 2 |
|  |  | 0: MNC length is 1 |
|  | Bit0~bit14 | MCC Information |

Example data:

  - Detailed explanation of terminal information

| Bit |  | Code meaning |
| --- | --- | --- |
| BYTE | Bit7 | 1: Oil and electricity disconnected |
|  |  | 0: Hybrid and electric connected |
|  | Bit6 | 1: GPS positioning |
|  |  | 0: GPS not positioned |
|  | Bit3～Bit5 | 100: SOS |
|  |  | 011: Low battery alarm |
|  |  | 010: Power failure alarm |
|  |  | 001: Vibration alarm |
|  |  | 000: Normal |
|  | Bit2 | 1: Connected to power and charging |
|  |  | 0: Not connected to a power source for charging |
|  | Bit1 | 1: ACC high |
|  |  | 0: ACC low |
|  | Bit0 | 1: Fortification |
|  |  | 0: Disarm |

  - Detailed explanation of alarm language

| Byte 1 | 0x00: Normal |
| --- | --- |
|  | 0x01: SOS |
|  | 0x02: Power failure alarm |
|  | 0x03: Vibration alarm |
|  | 0x04: Fence entry alarm |

|  |  | 0x05: Fence-out alarm |  |
| --- | --- | --- | --- |
|  |  | 0x06 Overspeed Alarm |  |
|  |  | 0x09 Displacement Alarm |  |
|  |  | 0x0A Entering GPS blind area alarm |  |
|  |  | 0x0B GPS blind zone alarm |  |
|  |  | 0x0C Power-on alarm |  |
|  |  | 0x0D GPS first positioning alarm |  |
|  |  | 0x0E External power low alarm |  |
|  |  | 0x0F External low power protection alarm |  |
|  |  | 0x10 Card replacement alarm |  |
|  |  | 0x11 Shutdown Alarm |  |
|  |  | 0x12 Flight mode alarm after external low battery protection |  |
|  |  | 0x13 Disassembly Alarm |  |
|  |  | 0x14 Door Alarm |  |
|  |  | 0x15 Low power shutdown alarm |  |
|  |  | 0x16 Voice-activated alarm |  |
|  |  | 0x17 Fake Base Station Alarm |  |
|  |  | 0x18 Cover open alarm |  |
|  |  | 0x19 Internal battery low power alarm |  |
|  |  | 0x20 Entering deep sleep alarm |  |
|  |  | 0x21 Reserved |  |
|  |  | 0x22 Reserved |  |
|  |  | 0x23 Fall Alarm |  |
|  |  | 0xFF ACC off |  |
|  |  | 0xFE ACC open |  |
|  | Byte 2 | 0x01 Chinese 0x02 English 0x00 No platform response required |  |

## Large file transfer

  - Used to transfer large files such as voice files

  - The terminal sends data to the server using the 8D protocol, and the server sends data to the terminal using the 90 protocol. The protocol formats are the same.

  - Because the terminal space is valid, the server needs to send data to the terminal through a series of handshakes to confirm that the terminal is available before transmission can proceed.

#### Terminal transfers files to server ( 8D )

  - The terminal sends to the server

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x8D |
| Informati on content | File Type | 1 | 0x00 Recording file (monitoring) 0x01 Recording file (SOS, voice control) 0x02 Intercom recording file 0x03 TCP command recording file upload |
|  | Total file length | 4 | Total length of transferred files |
|  | File error checking type | 1 | When the error check type is "00", use CRC check to transfer files When the error check type is "01", use MD5 to check the transferred file |
|  | File error checking | N | When the error check type is "00", the CRC check is used to transmit the file, and the result length is 2 bits. When the error check type is "01", the MD5 checksum is used to check the transmitted file, and the result length is 16 bits. |
|  | Starting position | 4 | Number of bytes at the start position of the transmission segmentation |
|  | Current content length | 2 | The length of data after the transmission segmentation starts |
|  | content | M | For the segmented data packets |
|  | Flag | N | The file type represents the type of the transferred file When the file type is 00 recording file (monitoring), this position is six bytes, which transmit the date and time when monitoring starts. The encoding method is the same as the positioning packet time format. When the file type is 01 recording file (SOS), this |

|  |  |  |  | position is two bytes, and the bytes are the same as the corresponding SOS, voice alarm package serial number When the file type is 02 recording file (intercom), this position is six bytes, the date and time when the transmission monitoring starts, the encoding method is the same as the positioning packet time format When the file type is 03 TCP command recording file upload, the unified upload service identification ID is used. The number of bytes is 6 bytes. The first 4 bits are the server flag bit and the last two bits are the transmission sequence number. |  |
| --- | --- | --- | --- | --- | --- |
|  | Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |  |
|  | Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |  |
|  | Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |  |

## Server reply

|  |  | len gth | Detailed explanation |
| --- | --- | --- | --- |
| Start bit |  | 2 | 0x79 0x79 |
| Packet length |  | 2 | Length = protocol number + information content + information sequence number + error check |
| Protocol Number |  | 1 | 0x8D |
| Informati on content | Receive status flag | 1 | Receiving normally 0x00 Receiving error 0x01 |
| Information sequence number |  | 2 | After power on, the serial number of each data sent is automatically increased by 1 |
| Error checking |  | 2 | The CRC-ITU value from "Packet Length" to "Message Sequence Number". If the receiver calculates a CRC error in the received message, it will be ignored and the data packet will be discarded (see Appendix 1 for the algorithm). |
| Stop bits |  | 2 | Fixed value, unified as 0x0D 0x0A |

| 三、 1. |  | appendix Appendix 1 CRC-ITU table lookup algorithm C language code snippet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| static |  | const U16 crctab16[] |  |  |  | = |  |  |  |  |  |
| { | 0X0000, |  | 0X1189, | 0X2312, |  | 0X329B, | 0X4624, | 0X57AD, |  | 0X6536, | 0X74BF, |
|  | 0X8C48, |  | 0X9DC1, | 0XAF5A, |  | 0XBED3, | 0XCA6C, | 0XDBE5, |  | 0XE97E, | 0XF8F7, |
|  | 0X1081, |  | 0X0108, | 0X3393, |  | 0X221A, | 0X56A5, | 0X472C, |  | 0X75B7, | 0X643E, |
|  | 0X9CC9, |  | 0X8D40, | 0XBFDB, |  | 0XAE52, | 0XDAED, | 0XCB64, |  | 0XF9FF, | 0XE876, |
|  | 0X2102, |  | 0X308B, | 0X0210, |  | 0X1399, | 0X6726, | 0X76AF, |  | 0X4434, | 0X55BD, |
|  | 0XAD4A, |  | 0XBCC3, | 0X8E58, |  | 0X9FD1, | 0XEB6E, | 0XFAE7, |  | 0XC87C, | 0XD9F5, |
|  | 0X3183, |  | 0X200A, | 0X1291, |  | 0X0318, | 0X77A7, | 0X662E, |  | 0X54B5, | 0X453C, |
|  | 0XBDCB, |  | 0XAC42, | 0X9ED9, |  | 0X8F50, | 0XFBEF, | 0XEA66, |  | 0XD8FD, | 0XC974, |
|  | 0X4204, |  | 0X538D, | 0X6116, |  | 0X709F, | 0X0420, | 0X15A9, |  | 0X2732, | 0X36BB, |
|  | 0XCE4C, |  | 0XDFC5, | 0XED5E, |  | 0XFCD7, | 0X8868, | 0X99E1, |  | 0XAB7A, | 0XBAF3, |
|  | 0X5285, |  | 0X430C, | 0X7197, |  | 0X601E, | 0X14A1, | 0X0528, |  | 0X37B3, | 0X263A, |
|  | 0XDECD, |  | 0XCF44, | 0XFDDF, |  | 0XEC56, | 0X98E9, | 0X8960, |  | 0XBBFB, | 0XAA72, |
|  | 0X6306, |  | 0X728F, | 0X4014, |  | 0X519D, | 0X2522, | 0X34AB, |  | 0X0630, | 0X17B9, |
|  | 0XEF4E, |  | 0XFEC7, | 0XCC5C, |  | 0XDDD5, | 0XA96A, | 0XB8E3, |  | 0X8A78, | 0X9BF1, |
|  | 0X7387, |  | 0X620E, | 0X5095, |  | 0X411C, | 0X35A3, | 0X242A, |  | 0X16B1, | 0X0738, |
|  | 0XFFCF, |  | 0XEE46, | 0XDCDD, |  | 0XCD54, | 0XB9EB, | 0XA862, |  | 0X9AF9, | 0X8B70, |
|  | 0X8408, |  | 0X9581, | 0XA71A, |  | 0XB693, | 0XC22C, | 0XD3A5, |  | 0XE13E, | 0XF0B7, |
|  | 0X0840, |  | 0X19C9, | 0X2B52, |  | 0X3ADB, | 0X4E64, | 0X5FED, |  | 0X6D76, | 0X7CFF, |
|  | 0X9489, |  | 0X8500, | 0XB79B, |  | 0XA612, | 0XD2AD, | 0XC324, |  | 0XF1BF, | 0XE036, |
|  | 0X18C1, |  | 0X0948, | 0X3BD3, |  | 0X2A5A, | 0X5EE5, | 0X4F6C, |  | 0X7DF7, | 0X6C7E, |
|  | 0XA50A, |  | 0XB483, | 0X8618, |  | 0X9791, | 0XE32E, | 0XF2A7, |  | 0XC03C, | 0XD1B5, |
|  | 0X2942, |  | 0X38CB, | 0X0A50, |  | 0X1BD9, | 0X6F66, | 0X7EEF, |  | 0X4C74, | 0X5DFD, |
|  | 0XB58B, |  | 0XA402, | 0X9699, |  | 0X8710, | 0XF3AF, | 0XE226, |  | 0XD0BD, | 0XC134, |
|  | 0X39C3, |  | 0X284A, | 0X1AD1, |  | 0X0B58, | 0X7FE7, | 0X6E6E, |  | 0X5CF5, | 0X4D7C, |
|  | 0XC60C, |  | 0XD785, | 0XE51E, |  | 0XF497, | 0X8028, | 0X91A1, |  | 0XA33A, | 0XB2B3, |
|  | 0X4A44, |  | 0X5BCD, | 0X6956, |  | 0X78DF, | 0X0C60, | 0X1DE9, |  | 0X2F72, | 0X3EFB, |
|  | 0XD68D, |  | 0XC704, | 0XF59F, |  | 0XE416, | 0X90A9, | 0X8120, |  | 0XB3BB, | 0XA232, |
|  | 0X5AC5, |  | 0X4B4C, | 0X79D7, |  | 0X685E, | 0X1CE1, | 0X0D68, |  | 0X3FF3, | 0X2E7A, |
|  | 0XE70E, |  | 0XF687, | 0XC41C, |  | 0XD595, | 0XA12A, | 0XB0A3, |  | 0X8238, | 0X93B1, |
|  | 0X6B46, |  | 0X7ACF, | 0X4854, |  | 0X59DD, | 0X2D62, | 0X3CEB, |  | 0X0E70, | 0X1FF9, |
| 0XF78F, 0XE606, 0XD49D, |  |  |  |  | 0XC514, 0XB1AB, 0XA022, |  |  |  | 0X92B9, 0X8330, |  |  |
| 0X7BC7, 0X6A4E, 0X58D5, |  |  |  |  | 0X495C, 0X3DE3, 0X2C6A, |  |  |  | 0X1EF1, 0X0F78, |  |  |
| }; |  |  |  |  |  |  |  |  |  |  |  |
| // Calculate the 16-bit |  |  |  |  | CRC of the given length |  |  |  | of data. |  |  |

U16 GetCrc16(const U8* pData, int nLength)

{

U16 fcs = 0xffff; // Initialization while(nLength>0){

fcs = (fcs >> 8) ^ crctab16[(fcs ^ *pData) & 0xff]; nLength--;

pData++;

}

return ~fcs; // invert

}

- Appendix 2 Business Flowchart